DArE: DECEMBER 17,2012 CLERK:_--'-M.:...:..:.A--'-'-R--'--Y'----B~E=_-:....:..rH...:......=:BA~1_

L.=::E...:...Y

MASSILLON CITY COUNCIL

CITY OF MASSILLON, OHIO

GLENN E. GAMBER, PRESIDENT

COUNCIL CHAMBERS LEGISLATIVE DEPARTMENT

ORDINANCE NO. 140 -2012

BY: ENVIRONMENTAL COMMITTEE

TITLE: AN ORDINANCE repealing certain Subsections of CHAPTER 925 "SEWERS GENERALLY" of the Codified Ordinances of the City of Massillon, Ohio, and enacting new Subsections to such chapter, and declaring an emergency.

NOW, THEREFORE, BE IT ORDAINED BY THE COUNCIL OF THE CITY OF MASSILLON, STATE OF OHIO, THAT:

Section I:

The existing Sections 925.01 PURPOSE; POLICY; DEFINITIONS, 925.12 SEWER CONNECTION PERMIT REQUIRED, FEE, 925.17 INSPECTION OF BUILDING SEWER FOR PERMIT, 925.20 GENERAL DISCHARGE PROHIBITIONS, 925.23 RIGHT OF REVISION, 925.25 SUPPLEMENTARY LIMITATIONS, 925.28 TREATMENT OF INDUSTRJAL WASTES, 925.30 ACCIDENTAL DISCHARGES, 925.32 WASTEWATER DISCHARGE PERMITS, 925.33 PRETREATMENT SCHEDULING REQUIREMENT, 925.34 PERMIT MODIFICATIONS, 925.35 PERMIT CONDITIONS, 925.37 LIMITATIONS ON PERMIT TRANSFER, 925.38 COMPLIANCE DATE REPORT, 925.39 PERIODIC COMPLAIANCE REPORTS, 925.40 MONITORING FACILITIES, 925.41 INSPECTION AND SAMPLING, 925.42 CONFIDENTIAL INFORMATION,

925.43 EMERGENCY SUSPENSION OF SERVICE AND DISCHARGE PERMITS, 925.44 REVOCATION OF PERMIT, 925.45 NOTIFICATION OF VIOLATION; ADMINISTRATIVE ADJUSTMENT, 925.46 SHOW CAUSE HEARING, 925.47 JUDICIAL PROCEEDINGS, 925.48 PRETREATMENT VIOLATION ENFORCEMENT RESPOI\JSE POLICY, 925.49 RIGHT OF APPEAL, 925.50 OPERATING UPSETS, 925.51 ADMINISTRATIVE PENALTIES, 925.54 RECORDS RETENTION, 925.55 FEES, 925.56 RIGHT TO CONTRACT WITH INDUSTRY, 925.60 BASELINE MONITORING REPORT, 925.61 SIGNATORY REQUIREMENTS FOR INDUSTRIAL USER REPORTS AND CERTIFICATIONS and APPENDIX A -PRIORITY TOXIC POLLUTANTS, of CHAPTER 925 of the Codified Ordinances of the City of Massillon, Ohio, be and hereby are repealed and deleted.

· . -2-ORDINANCE NO. 140 -2012

Section 2:

New revised Sections 925.01 PURPOSE; POLICY; DEFINITIONS, 925.12 SEWER CONNECTION PERMIT REQUIRED, FEE, 925.17 INSPECTION OF BUILDING SEWER FOR PERMIT, 925.20 GENERAL DISCHARGE PROHIBITIONS, 925.23 RIGHT OF REVISION, 925.25 SUPPLEMENTARY LIMITATIONS, 925.28 TREATMENT OF INDUSTRIAL WASTES, 925.30 ACCIDENTAL DISCHARGES, 925.32 WASTEWATER DISCHARGE PERMITS, 925.33 PRETREATMENT SCHEDULING REQUIREMENT, 925.34 PERMIT MODIFICATIONS, 925.35 PERMIT CONDITIONS, 925.37 LIMITATIONS ON PERMIT TRANSFER, 925.38 COMPLIANCE DATE REPORT, 925.39 PERIODIC COMPLAIANCE REPORTS, 925.40 MONITORING FACILITIES, 925.41 INSPECTION AND SAMPLING, 925.42 CONFIDENTIAL INFORMATION,

925.43 EMERGENCY SUSPENSION OF SERVICE AND DISCHARGE PERMITS, 925.44 REVOCATION OF PERMIT, 925.45 NOTIFICATION OF VIOLATION; ADMINISTRATIVE ADJUSTMENT, 925.46 SHOW CAUSE HEARING, 925.47 JUDICIAL PROCEEDINGS, 925.48 PRETREATMENT VIOLATION ENFORCEMENT RESPONSE POLICY, 925.49 RIGHT OF APPEAL, 925.50 OPERATING UPSETS, 925.51 ADMINISTRATIVE PENALTIES, 925.54 RECORDS RETENTION, 925.55 FEES, 925.56 RIGHT TO CONTRACT WITH INDUSTRY, 925.60 BASELINE MONITORING REPORT, 925.61 SIGNATORY REQUIREMENTS FOR INDUSTRIAL USER REPORTS AND CERTIFICATIONS and APPENDIXA-PRIORITY TOXIC POLLUTANTS, of CHAPTER 925 of the Codified Ordinances of the City of Massillon, Ohio, are hereby enacted as set forth on the attached:

(SEE ATTACHED EXHIBIT "A")

Section 3

The Clerk of Council is authorized to correct any typographical errors discovered herein during or after the pendency or passage of this ordinance. The Clerk of Council is further authorized, in conjunction with the Law Department and the Council President to correct any ministerial or de minimis errors that do not substantially alter the intended results or numerical total sums of this ordinance, during or after the pendency or passage of this ordinance. Corrected copies are to be sent to all official recipients.

Section 4:

That this Ordinance is hereby declared to be an emergency measure necessary for the preservation of the health, safety and welfare of the community, and for the further reason that the proposed changes are minor grammar corrections throughout the chapter. Provided it receives the affirmative vote of two-thjrds of the elected members to Council, it shall take effect and be in force immediately upon its passage and approval by the Mayor. Otherwise, it shall take effect and be in force from and after the earliest period allowed by law.

ATTEST:

APPROVED~ ,yVt 0J(}/c3

-PERRY, MAYOR

I

925.01 PURPOSE; POLlCY; DEFJNITIONS

(a) The purpose of this chapter is to set forth uniform requirements for dischargers into the City's wastewater collection and treatment systems, and to enable the City to protect public health in conformity with all applicable local, state, and federal laws relating thereto.

(b) The objectives of this chapter are:

(1) To prevent the introduction of pollutants into the City wastewater system which will interfere with the normal operation of the system or contaminate the resulting Municipal sludge;

(2) To prevent the introduction of pollutants into the City wastewater system which do not receive adequate treatment in the City's wastewater treatment facilities, and which will pass through the system into receiving waters or the atmosphere or otherwise be incompatible with the system;

(3) To improve the opportunity to recycle and reclaim wastewater and sludge from the system. (Ord. 69-1991. Passes 4-15-91.)

(c) Definrtion. 	Unless the context specifically indicates otherwise, the following terms and phrases, as used in this chapter shall have the meanings hereinafter designated:

(1) "Act" or "the Act" means the Federal Water Pollution Control Act, also known as the Clean Water Act, as amended, 33 U.S.C. 1251, et seq,

(2) "Approved Authority" means the Director in an NPDES state with an approved State Pretreatment Program and the Administrator of the EPA in a nonNPOES state or NPDES state without an Approved State Pretreatment Program.

(3) "Best M81nagement Practices (8MP)" means schedules of actEv1Hes, prohibitions of practices, maintenance procedures, and other management pradices to implement the prrohibitions listed in OAG 3745·3a04 -General Discharge Prohibitions. BMpls also include treatment requirements, operating procedures, and practices to control plant site runoff, spillage or leaks, sludge or waste disposa~, or drainage from raw materials storage.

(4) "Biochemical Oxygen Demand (BOD)" means the quantity of oxygen utilized in the biochemical oxidation of organic matter under standard laboratory procedure, five days at 20 C expressed in terms of weight and concentration (milligrams per liter (mg/L)).

(5) "Building Sewer" means a sewer conveying wastewater from the premises of a User to the POTW.

(6) "Bypass" means the intentional diversion of waste streams from any portion of an Industrial User's treatment faciljty.

(7) "Categorical Standards" means National Categorical Pretreatment Standards or Pretreatment Standard specifying quanmies or concentrations of poUUltants or pollutant properties which may be discharged or introduced into the sewage treatment system by specific undustria! Users.

(8) "City" means the City of Massillon or the City Council of Massillon.

(9) "Compatible Pollutant" indudes biochemical oxygen demand, suspended solids, pH, fecal coliform bacteria, and e. coli bacteria, plus additional pollutants identified by the City's National Pollutant Discharge Elimination S}fstem (I\lPDES) Permit, providing the City's treatment system is designed to treat such pollutants, and in fact does remove such pollutants to a substantial degree.

(10) 	"Cooling Water" or "Industrial Cooling Water" means the unpolluted water discharged from any system of condensation, air concHtioning cooHng, refrigeration, or to which the only pollutant added is heat, or other similar use which meet the criteria established by the OEPA for effluents dtscharged to watercourses at Massmon, Ohio.

(11) 	HDaHy Maximum" means the arithmetic average of al! effluent samples coJiected during a calendar day.

(12) 	"Daily Maximum Limit" means the maximum allowable discharge limit of a pollutant during a calendar day. Where Daily Maxnrnurn Limits are expressed in units of mass, the daily discharge is the total mass discharged over the course of the day. Where Daily Maximum Limits are expressed in terms of a concentration, the daily dlischarge is the adthmetic average measurement of the pollutant c061lcentration derived from all measurements taKen that day.

(13) 	~Djrect Discharge" means the discharge of treated or untreated wastewater directly to the waters of the State of Ohio.

(14) 	"Environmental Protection Agency or EPA" means the U.S Environmental Protection Agency, or where appropriate the term may also be used as a designation for the Administrator or other duly authorized official of such agency.

(15) 	"Grab Sample" means a sample which is taken from a waste stream on a one-time basis with no regard to the flow in the waste stream and without consideration of time.

(16) 	"Holding Tank Waste" means any waste from holding tanks such as vessels, chemical toilets, campers, trailers, septic tanks, and vacuum-pump tank truck.

(17) 	"Incompatible Pollutant'S means any ponutaJnt which is not at compatible pollutant as defined in subsection 925.01 (c}(9) hereof.

(18) 	"Indirect Discharge" means the discharge or the introduction of nondomestic pollutants from any source regulated under section 307(b) or (c) of the Act, (33 U.S.C, 1317). Into the POTW (including holding tank waste discharged into the system).

(19) 	"Industrial User (I U)" means a source of lndirect Discharge which does not constitute a "discharge of pollutants" under regulations issued pursuant to section 402, of the Act. (33 U.S.C. 1342).

(20) 	"tnstantaneoUis limit" means the maximum concentration of a poHutant elBowed to be discharge at any time, determined from the analysis of any discrete or composited sample coUected, independent of the industrial flow rate and the durat~on of the sampEing event.

(21) 	"Interference" means a discharge which, alone or in conjunction with a discharge or discharges from other sources, both:

A. 	Inhibits or disrupts the POTW, Its treatment processes or operations, or its sludge processes, use or disposal: and

B. 	Therefore is a cause of a violation of any requirement of the POTW's NPDES permit (including an increase in the magnitude or duration of a violation) or of the prevention of sewage sludge use or disposal in compliance with the following statutory provisions and regulations or permits issued there under (or more stringent State or local regulations): Section 405 of the Clean Water Act, the Solid Waste Disposal Act (SWDA) (including Title II, more commonly referred to as the Resource ConseNatlon and Recovery Act (RCRA), an including State regulations contained in any State sludge management plan prepared pursuant to Subtitle 0 of the SWDA), the Clean Air Act, the Toxic Substances Control Act, and the Marine Protection, Research and Sanctuaries Act. Section 403.3(i).

(22) 	"Local limit" means specific discharge iimits developed and enforced by the City upon industrial or commercial facifitnes to implement the genera! and specific discharge prohibitions listed in 40 CFR 403.5(a)(1) and (b).

(23) 	"Manager" means the person designated by the City to supervise the operation of the publicly owned treatment works and who is charged with certain duties and responsibilities by this chapter, or his duly authorized representative.

(24) 	"Month~y Average" means the sum of aU "daily discharges" measured during a calendar month divided by the number of "daily discharges" measured during the month.

(25) 	llMonthly Average limit" means the highest allowab!e average of "daily discharges" over a ca~endar month, cakuiClited as the sum of all "daily discharges" measured during a calendar month divided by the number of "daftly discharges" measured during that month.

(26) 	"National Categorical Pretreatment Standard or Pretreatment Standard" means any regulation containing pollutant discharge limits promulgated by the EPA in accordance with section 307 (b) and (c) of the Act (33 U,S.C.1347) which applies to a specific category of Industrial Users and that appear in 40 GFR Chapter 1, Subchapter N, Parts 405-471.

(27) 	A. "New Source" means any building, structure, facility or installation from which there is or may be a discharge of pollutants, the construction of which commenced after the publication of proposed Pretreatment Standards under Section 307{c) of the Act which will be applicable to such source if such Standards are thereafter promulgated in accordance with that section provided that:

i. 	The building, structure, facility or installation is constructed at a site at which no other source is located: or

The building, structure, facility or installation totally replaces the process or production equipment that causes the discharge of pollutants at an existing source:

The production or wastewater generating processes of the building, structure, facility or installation are substantiaHy independent of an existing source at the same site. In determining whether these are substantially independent, factors such as the extent to which the new facility is integrated with the existing plant, and the extent to which the new facility is engaged in the same general type of activity as the existing source should be considered.

B. Construction on a site at which an Existing Source is iocated results in a modification rather than a New Source if the constn.ndion does not create a new building, structure, facility, or installation meeting the criteria of Section (A)(1) or (2) above but otherwise alters, replaces, or adds to existing process or production equipment.

C. Construction of a New Source as defined under this paragraph has commenced if the owner or operator has:

1. Begun, or caused to begin, as part of a continuoUls onsite construction program.

any placement, assembly, or installation of facilities or equipment; or

significant site preparation work inducing clearing, excavation, or removal of exnsting buildings, structures, or facilities which is necessary for the placement, assembly, or installation of new source facilities or equipment; or

2. Entered into a binding contractual obHgation for the purchase of fadlities or equipment which are intended to be used! in its operation withnn a reasonable time. Options to purchase or contracts which can be terminated or moolBfied without substantial loss, and contracts for feasobility, engineering, and design studies do not constitute a contractual obligation under this paragraph.

(28) 	"National Pollution Discharge Elimination System or NPDES Permit" means a permit issued pursuant to Sectjon 402 of the Act (33 U.S.C. 1342).

(29) 	"Normal Sarength Sewage or Waste" means sewage or waste used for the purpose of determining sl1rchalfge means. Normal strength sewage or waste means having an average dalHy suspended so~ids concentration of not more than 300 mg/l, an average daily BOD concentration of not more than 300 mg/L., and not containing ai'll' of the characteristics in excess of the limitations as prohcbited and estabUshed by Section 925.20.

(30) "OEPA" means Ohio Environmenta! Protection Agen.cy.

(31) 	"on and Grease" means fats, grease, oil, wax, or other material which is extractable from an acidified samp~e of a waste by hexane or other designated solvent.

(32) 	"Pass-through" means a discharge which exits the POTW into water of the United States in quantities or concentrations which, alone or in conjunction

with a discharge or discharges from other sources, is a cause of a violation of

any requirement of the POTW's NPDES permit (including an increase in the

magnitude or duration of a violation). Section 403.3(n) of the General

Pretreatment Regulation (40 CFR, Part 403).

(33) 	"Person" means individual, partnership, co partnership, firm, company, corporation, association, joint stock company, trust, estate, governmental entity or any other legal entity, or their legal representative, agents, or assigns. The masculine gender shall include the feminine, the singular shall include the plural where indicated by the context.

(34) 	"pH" means the logarithm (base 10) of the reciprocal of the concentration of hydrogen ions expressed in grams per liter of solution.

(35) 	"Pollution" means the man-made or man-induced alteration of the chemical, physical, biological, and radiological integrity of the water.

(36) 	"Pollutant" means any dredged soil, solid waste, incinerator residue, sewage, garbage, sewage sludge, munitions, chemical wastes, biological materials, radioactive material, heat, wrecked or discharged equipment, rock, sand, cellar dirt, industrial, municipal, and agricultural waste, discharged into water.

(37) 	"Pretreatment or Treatment" means the reduction of the amount of pollutants, the elimination of pollutants, or the alteration of the nature of pollutant properties in wastewater to a less harmful state prior to or in lieu of discharging or othervvise introducing such pollutants into a POTW. The reduction or alteration can be obtained by physical, chemical, or biological processes, or process changes other means, except as prohibited by 40 CFR, Section 403.6(d)

(38) 	"Pretreatment Requirements" means any substantive or procedural requirement related to pretreatment, other than a National Pretreatment Standard imposed on an industrial user.

(39) 	"Publically Owned Treatment Works (POTVV)" means a treatment works as defined by Section 212 of the Act, (33 U.S.C. 1292) which is owned in this instance by the City. This definition includes any sewers that convey wastewater to the POTW treatment plant, but does not include pipes, sewers, or other conveyances not connected to a facility prOViding treatment. For the purposes of this chapter, "POTW" shall include any sewers that convey wastewater to the POTW from persons outside the City who are, by contract or agreement with the City, users of the City's POTW.

(40) 	POTW Treatment Plant" means that portion of the POTW designed to provide treatment to wastewater.

(41) 	"Safety-Service Director" means the Director of Public Safety and Service of the City of Massillon or his duly authorized representative.

(42) 	"Septic Tank Waste" means any sewage from holding tanks, such as vessels, chemical toilets, campers, tra.~iers, amd septic tanks.

(43) 	"Sewage" means human excrement amd gray water (household showers, dishwashing operation, etc.)

(44) "Shall" is mandatory: "May" is permissive.

(45) "Significant Industrial User"

A. 	Except as provided in part B of this subsection, the term Significant Industrial User includes:

All industrial users subject to categorical pretreatment standards; and

Any other industrial user that: discharges an average of 25,000 gallons per day or more of process wastewater to the POTW; contributes a process waste stream which makes up five percent (5%) or more of the average dry weather hydraulic or organic capacity of the POTW treatment plant; or has a reasonable potential, in the opinion of the Safety-Service Director, to adversely affect the POTW's operation or for violating any pretreatment standard or requirement.

B. 	The Safety-Service Director may at any time, on its own initiative or in response to a petition received from an industrial user, determine that a noncategorical industrial user has no reasonable potential to adversely affect the POTW's operation or for violating any pretreatment standard or requirement.

C. 	The City may determnne that an ~l1Idustria! User subject to categorical pretreatment standards is a Non-Significant Categorical! ~ndustriai! User rather than a Significant industrial User on a finding that the Industrial! User never discharges more than 11 00 gallons per aiay (gpd) of total categorical Walstewater (excluding sanitary, non~contadcooling and boiler blowdown wast.ewater, unless specifically induded in the Pretreatment Standard) and the following conditions are met:

The industrial User, prior to the City's findingl has consistenUy complied with aH app~icab!e categorical Pretreatment Standards and Requirements:

The ~ridustrial User annually submits the certification stafrement required in Section 925.61, together with any additional information necessary to support the certificatoon statement; and

The undustrial User never dtscharges any untreated concentrated wastewater.

D. 	Upon a finding tha~ a User meeting the criteria in Subsection A of thIS part has no reasonable potentoa! for adversely affecting the POTVV's operation or for vio~ating any Pretreatment Standard or Requirement, the City cay at any time, on its own 6nitiative or on response to a petition received from an ~ndustrial User, aJnd in accordance with procedures in 40 CFR 4Q3,8(f)(6}, determine that such User should not be considered Cd Significa01lt industrial User.

(46)Slug" means any pollutant, inclUding oxygen demanding pollutants released in a single m<traordinary discharge episode of such volume or strength as to cause interference to the City's vvastewater treatment plant.

(46) 	"Slug LoadDog or Slug Discharge" means any discharge at a flow rate or concentration which could cause a vio!ation of the prohibited discharge standards in Section 925.20 of the Ordinance. A slug discharge is any discharge of a nonroUitine, episodic nature, irsciuding but not limited to an accidental spiH or a nOrilcustomary ba~ch d~scharge which has a reasonable potential to cause i61terference or pass-through, or in any other way violate the POTW's loca! limits or permit conditions.

(47) "State" means State of Ohio.

(48) 	"Standard IndustriaJ Classification (SI C)" means a classification pursuant to the Standard Industrial Classification Manual issued by the Executive Office of the President, Office of Management and Budget, 1972.

(49) 	"Storm Water" means any flow occurring during or following any form of natural preclpitation and resulting therefrom, inclUding snowmelt.

(50) 	"Surcharge" means the fee in adcHtion to the service charge which is ievied on those persons whose wastes are greater in strength than the concentration values established as representative of normal strength.

(51) 	"Suspended Solids means the total suspended matter that floats on the surface of, or is suspended in water, wastewater or other liquids, and which is removable by laboratory filtering

(52) 	"Toxic Pollutants" means any pollutant or combination of pollutants listed as toxic in regulations promulgated by the Administrator of the Environmental Protection Agency under the provisions of CWA 307(a) or other Acts.

(53) 	"Upsets" means an exceptional incident which creates a temporary noncompliance with permit limits due to the impacts of the incoming waste characteristics on the POTW treatment processes.

(54) 	"User" means any person who contributes, causes, or permits the contribution of wastewater into the City's POTW.

(55) 	"Wastewater" means the liquid and water-carried industrial or domestic wastes from dwellings, commercial buildings, industrial facilities: and institutions, together with may be present, whether treated or untreated, which IS contributed into or permitted to enter the POTW.

(56) 	"Waters of the State" means all streams, lakes, ponds, marshes, watercourses, waterways, wells, springs, reservoirs, aquifers, irrigation systems, drainage system and all other bodies or accumulations of water, surface or underground, natural or artificial, public or private, which are contained within, flow through, or border upon the State or any portion thereof.

(57) "Wastewater Discharge Permit" means as set forth in Section 925.32.

(d) Abbreviations. The following abbreviations shall have the designated meanings:

| ANSI: | American National Standards Institute |  |
| --- | --- | --- |
| ASTM: | American Society for Testing and Materials |  |
| BM?: | Best Management Practnce |  |
| BMR; | BaseHne Monitoring Report |  |
| BOD: | Biochemical Oxygen Demand |  |
| CFR: | Code of Federal Regulations |  |

COD: EPA:

gpd:

IU:

L: mg: mg/L: NPDES: POTW: SIC: SIU: SNC: SWDA: TSS: USC: WPCF:

Chemical Oxygen Demand Environmental Protection Agency Gallons per day Industrial User Liter Milligram Milligram per liter NatJonal Pollutant Discharge Elimination System Publicly Owned Treatment Works Standard Industrial Classification Significant industrial User Significant Non~compHance Solid Waste Disposal Act, 42, U.S.C. 6901, e1. Seq. Total Suspended Solids United States Code

Water PoHution Control Federation

(Ord. 69-1991. Passed 4-15-91.)

925.12 SEWER CONNECTION PERMIT REQUIRED, FEE

(a) No authorized person shall uncover, make any connections with or opening into, use. alter or disturb any public sewer, or appurtenance without first obtaining a written permit from the City Engineer.

(Ord. 69-1991. Passed 4-15-91.)

(b) The permit application shall be supplemented by any plans, specifications or other information considered pertinent in the judgment of the City Engineer. A permit and inspection fee of one hundred dollars ($100.00) for a residential building sewer permit and two hundred dollars ($200.00) for a commercial building sewer permit and for an industria! waste permit shall be paid to the City Engineer at the time application is filed. A permit fee for the repair of an eXisting sewer shall be fifty dollars ($50.00).

(Ord. 55-2004. Passed 4-19-04.)

(b) The permit application shall be supplemented by any plans, specifications or other information considered pertinent in the judgment of the City Engineer. A permit and inspection fee of one hundred fifty dollars ($150.00) for a residential building sewer permit and two hundred fifty dollars ($250.00) for a commercial bUilding sewer permit and for an industrial waste permit shaH be paid to the City Engineer at the time application is filed. A permit fee for the repair of an existing sewer shall be seventy-five dollars ($75.00). (Ord. 46-2012. Passed 6-4-12.)

(c) Provtded, however, no new permit will be issued when it has been determined by the City EngB!f1eer or Plant Manager of the Wastewater Treatment System Plant that there does not exist sufficient capacity in the Wastewater Treatment System P~ant including collecting, pumping, treating and disposing of wastes as may be discharged by the applicant of such new permit.

(Ord.69-1991. Passed 4-15-91.)

(d) At the time when the permft and inspection fee IS secured per subsection (b) hereof, a capacity charge for new connections shall apply and be calculated per the following table. Once the capacity charge is calCUlated, it shall be payable at the time the permit and inspection fee [s acquired.

| USER CLASS |  |  | EDUVALUE | CAPACITY CHARGE |  |
| --- | --- | --- | --- | --- | --- |
| Single Family Unit |  |  | 1.00 | $1,000.00 $2,000.00 |  |
| Du[)lex Residential |  |  | 2.00 |  |  |
| Each Addllional Resldenllal Unit |  |  | 1.00 | $1,000.00 |  |
| Mobile Homes and House Trailers |  |  | 1.00 | $1,000.00 |  |
| Church |  |  | 0.16 | $ 160.00 $ 670.00 $ 290.00 |  |
| ToilellUrinal-Each |  |  | 0.67 |  |  |
| Food Prep/Bar Sink (Per BOWl) |  |  | 0.29 |  |  |
| Hair Rinse Sink -Each |  |  | 0.16 | $ 160.00 $ 170.00 $1,140.00 |  |
| Slop Sink -Each |  |  | 0.17 |  |  |
| Hotel/Motel |  |  | 1,14 |  |  |
| Hotel/Motel -Each Bedroom |  |  | 0.17 | $ 170.00 $ 970.00 $ 170.00 iii 140.00 $ 20.00 |  |
| Rooming House |  |  | 0.97 |  |  |
| Rooming House· Each Bedroom |  |  | 0.17 |  |  |
| Hospitals/Nursing Homes (Per Employee/Patient) | 0.14 |  |  |  |  |
| Sports Facility/Arena (Per 100 Seats) | 0.02 |  |  |  |  |
| Laundromat or Coin Operated Machine |  |  | 0.59 | $ 590.00 |  |
| Commercial Laundries (8y Pound) | 0.07 | $ 70.00 |  |  |  |
| Car Washes· Each Single Slall Manual |  |  | 1.19 | $ 1.190.00 |  |
| Car Washes -Each Single SialI Automatic |  |  | 3.96 | $ 3,950.00 |  |
| Car Washes· Each Drive-Through Automatic |  |  | 25.00 | £25,000.00 |  |
| Schools, Public-Private-Day Care (Per Studenl and Employee) |  |  | 0.04 | $ 40.00 $ 120.00 |  |
| Hand Sink· Each |  |  | 0.12 |  |  |
| Showers -Each |  |  | 0.43 | :; 430.00 $ 170.00 |  |
| Induslrial Employee |  |  | 0.17 |  |  |

(Ord, 46-2012. Passed 6-4-12.)

925,17 INSPECTION OF BUILDING SEWER FOR PERMIT

The applicant for the building sewer shall notify the City Engineer Plant Manager when the building sewer is ready for inspection and connection to the public sewer, The connection shall be made under the supervision of the CBty Engineer Pffiffi Manager or his representative.

(Ord.69-1991. Passed 4-15-91,)

925.20 GENERAL DISCHARGE PROHIBITIONS

No discharger shall contribute or cause to be discharged, directly or indirectly, any of

the following described substances into the City wastewater collection and treatment

system:

(a) Any pollutants which by reason of their nature or quantity are, or may be, sufficient either alone or by interaction to create a fire or explosion hazard in the wastewater collection system and/or treatment plant including but not limited to, waste streams with a closed cup flashpoint of less than 140 degrees Fahrenheit or 60 degrees Centigrade using the test method specified in 40 CFR 261.21.

(b) Solid or viscous substances which will or may cause obstruction to the flow in a sewer or other interference with the operation of the wastewater system. (Ord. 691991. Passed 4-15-91.)

(c) Any wastewater having a pH less than 6.0 S.U. or higher than 10 S.U" or having any other corrosive property capable of causing damage or hazard to structures, equipment or personnel of the system.

(Ord. 56-1999. Passed 4-5-99.)

(d) Any wastewater containing toxic pollutants in sufficient quantity, either singly or by interaction to injure or interfere with any wastewater treatment process, constitute a hazard to humans or animals, or to exceed the limitation set forth in any applicable Categorical Pretreatment Standards. A toxic pollutant shall include but not be limited to any pollutant identified in the Toxic Pollutant List set forth in Appendix A hereto.

(e) Any noxious or malodorous liqUids, gases, or solids which either singly or by interaction are capable of creating a pUblic nuisance or hazard to life or are sufficient to prevent entry into the sewers for their maintenance and repair.

(f) Any substance which may cause the City wastewater treatment plant effluent or treatment residues, sludges, or scums to be unsuitable for reclamation and reuse or to interfere with the reclamation process.

(g) Any substance which will cause the City's wastewater treatment plant to violate its National Pollutant Discharge Elimination System (NPDES) and/or other Disposal System Permits.

(h) Any substance with objectionable color not removed in the treatment process, such as, but not limited to, dye wastes and vegetable tanning solutions.

(i) Any wastewater having a temperature which will inhibit biological activity in the City's wastewater treatment plant resulting in interference; but in no case, wastewater with a temperature at the introduction into the City's wastewater treatment plant which exceeds 40 degrees Celsius or 104 degrees Fahrenheit.

U) Any slugload, which means any pollutant, including oxygen demanding pollutants released in a single extraordinary discharge episode of such volume or strength as to cause interference to the City's wastewater treatment plant.

(k) Any wastewater containing any radioactive wastes or isotopes of such half life or concentration as exceed limits established by the City in compliance with

applicable State or Federal regulations.

(I) Any wastewater which causes a hazard to human ~ife or creates a public nuisance.

(m) Any water or wastes containing free oils, emulsifed oils, and grease exceeding an average of 100 parts per million of either soluble matter.

(n) Any petroleum oil, nonbiodegradable cutting oil or products of mineral oil origin discharged In amounts which cause pass through or interference at the wastewater treatment plant.

(0) Pollutants which result in the presence of toxic gases, vapors, or fumes within the wastewater collection system and/or treatment plant in a quantity that may cause acute worker health and safety problems

(p) Any garbage that has not been properly shredded.

(q) Any waters or wastes containing suspended solids of such character and quantity that unusual attention or expense is required to handle such materials at the wastewater treatment plant, or having a chlorine demand greater than 30 parts per million.

(r) Under no conditions will the discharge of concentrated plating baths or acid pickling liquor whether neutralized or not be permitted to the sanitary sewer system.

(s) Pollutants, including oxygerH~ernandingpollutants (BOD, etc.), released in a discharge at a flow rate and/or pollutant concentration which, either sing~y, or by interaction with other pollutants, wm cause interference w~th the POTW.

(t) Trucked or hauled pollutants, except at discharge ponrlts designated by the Plant Manager in accordance with Section 925.26 of this ordinance.

(u) Storm water, surface water, ground water, artesian well water, roof runoff, subsurface drcllin8lgel sv\dmming pool dra~lnlage, condensate, dleionized water, noncontact cooling water, and lllnpoliuted wastewater, unless specifically authorized by t.he Plant. Manager.

(v) SludJges, screenings, or o~her residues from the prerreatment of industrial wastes.

(w) Medical wastes, except i31S specifica~ly authorized by the Plant Manager in an individual wastewater discharge permit.

(x) Wastewater causing, aione or in coniursc~iorr:with other sourcest the treatment plant's emuent to fait toxicity test

(y) Detergents, surface~active agents, or other substances which might cause excessive foaming in the POTIiV

(z) Wastewater causing two successive readings on an explosion hazard meter at the point of discharge nnto the POTW, or at any point in. the system, be more than five percent (%) of the lower Explosive Umit (LEl) of the meter flIor shail any single reading be more than 10 percent (%) of the Lower ExplosDve limit (LEL) of the meter. (Ord. 69-1991. Passed 4-15-91.)

925.23 RIGHT OF REVISJON

:rhe City reserves the right to amend this chapter to provide for more stringent limitations or requirements on discharges to the City's 'Nactewater treatment piant where deemod nococsa!)'.

The City reserves the right to establish, by ordonance or in individual wastewater discharge permits, more stringent Standards or Requirements 0[11 discharges to the ponfl! consistent with the purpose of this ordinance.

(Ord. 69-1991. Passed 4-15-91.)

925.25 SUPPLEMENTARY LIMITATIONS

(a) No discharger shall discharge wastewater containing concentrations or loadings of pollutants in excess of those specified in its wastewater discharge permIt. Also, no discharger shall discharge other toxic pollutants listed in Appendix A, but not contained in the wastewater discharge permit, unless specifically authorized to do so in a revised permit. In any event, such discharges shall meet categorical pretreatment standards where applicable.

(b) All Industrial Users shall promptly notify the Safety Service Director Plant Manager in advance of any substantial changes in the volume or character of pollutants in thejr discharge, including the listed or characteristic hazardous wastes for which the Industrial User has submitted initial notification under 40 CFR 403.12(p).

(c) VVhenever deemed necessary, the Plant Manager may require Industrial Users to restrict their dfscharge during peak flow periods, designate that certain wastewater by discharged on!y into specific sewers, relocate andlor consolidate points of discharge, separate sewage wastewstreams from industria! wastestreaml and such other conditions as may be necessary to protect the POW\! ali!ld determine the indlustr~aJl User's compiiCijilCe with the requirer'-lents of this ordinance.

(d) The Plant Manger may require any person discharging into the POTVV to instaH and maintain, on theor property and at their expense, a suitable storage and flow-contro! facility to ensure equailzatiotl of flow. An individual W81steWaiter discharge permit may be issued solely for flow equalization.

(e) industrial Users witth the potential to discharge f!ammab~e substances may be required to insta!! and maintain an approved combustible gas dietection meter.

(Ord.69-1991. Passed 4-15-91.)

925.28 TREATMENT OF INDUSTRIAL WASTES

(a) The admission into the public sewers of any waters or wastes having:

(1) A 5-day biochemical oxygen demand greater than 300 parts per mJllion by weight; and/or

(2) More than 300 parts per million by weight of suspended solids; and/or

(3) Any quantity of substances with the characteristics described in Sections

925.20 and 925.25; and/or

(4) An average daily flow greater than 50,000 gallons per day;

shall be subject to the review and approval of the Safety-Service Director and the Plant Manager of the wastewater treatment plant.

(b) The Safety~Service Director and the Plant Manger Superintendent of the wastewater treatment plant shall have the right to reject any and all wastes which, in their opinion, may be harmful to or exceed capacities of the wastewater treatment system. Where necessary. in the opinion of the Safety-Service Director and the Plant Manager of the wastewater treatment plant, the owner shall provide at his expense such preliminary treatment as may be necessary to:

(1) Reduce the biochemical oxygen demand to 300 parts per million and the suspended solids to 300 parts per million by weight;

(2) Reduce objectionable characteristics or constituents to within the maximum limits provided for in Sections 925.20 and 925.25; or

(3) Control the quantities and rates of discharge of such waters or wastes. Detailed p~ans. specifications, operating procedures, and any other pertinent Information relating to proposed treatment facilities shall be submitted for the approval of the City Engineer, Safety-Service Director 9f and the Plant Manager of the wastewater treatment plant. The review of such plans and operating procedures shall in no way relieve the User fmm the responsibility of modifying such facilities as necessary to produce a discharge acceptBlb~e to the City under provisions of this ordinance. No construction of such facilities shall be commenced until such approval is obtained in writing.

(c) The discharge of industrial wastes with concentrations exceeding the "normal strength" limitations may be permitted when, in the opinion of the Safety-Service Director and the Plant Manager of wastewater treatment plant, it is determined that such wastes can be satisfactorily treated without adverse effects to the collection and/or treatment systems. The industrial waste mentioned in this section must be a biologically degradable wastewater constituent capable of being removed by the existing wastewater treatment plant. Priority pollutants do not fall within this category.

The discharge of such wastes exceeding "normal strength" shall be subject to and require a surcharge to compensate the City for all costs associated with the collection,

conveyance, treatment, and final disposal for all excess concentrations as provided for in Section 929.12 and/or Section 937.10. (Ord.69-1991. Passed 4-15-91.)

(d) The discharge of industrial waJstes with concentrations containing greater than the following limits require review and! approval of the City Engineer and Plant Manager before discharge. Thts approval may be outiined in the permit to discharge> or added as an addendum.

Tota! Phosphorus 8 mg/L Total Nitrogen 10 mg/l Sulfate 150 mg/L Dissolved Sulfide 0.1 mg/l Hydrogen Suifllde iO mg/L

(atmospheric) Total Dissolved Sorrids 1500 mg/l

925.30 ACCIDENTAL DISCHARGES SLUG LOADING

(a) Each disoharger shall provide protection from accidontal dischaFgo of prol4ibited or regulated materials or substances established by this ohapter. Where necessary, facilities to prevent accidental discharge of prohibited materials shall be provided and maintained at the discharger's 006t and expense. Detailed plans shmving facilities and operating prooedures to provide this protection sl4all be submitted to the Manager of the w-astm&Jater treatment system for review, and shall be approved by the Manager before construction of the faoility. Reviow and approval of such plans and operating procedures by the Managor shall not relieve the discharger from the responsibility to modify its facility as necessary to meet tho requlrements of this chaptor.

(b) Dischargors shall notify the Manager immediately upon the occurrence of a "slugload" or acci61ental dischaFge of substanoos prohibited by this chapter. The notification shall inclu61e location of discharge, date, and time thereof, type of ,,·vaste, concentration and volume, and corrective actions. Any discharger who 6lischarges a slugload of prohibited materials shall be liable for any expense, loss, or damage to the City's 'Nastolll3ter treatment plant in addition to tho amount of any fines imposed on the City on account thereof under State or Federal law.

(0) Signs shall be permanently posted in conspicuous places on discharger's premises advising employees ",','hom to call in tho event of a slug or accidental discharge. Employers shall lnstruct all employees \:vho may causa or discover such a discharge with respect tc emergency notification procedure.

SLUG DISCHARGE CONTROL PROGRAM

(a:) Each User shall provnde protection from slug doscharges of restricted materi@ls or other 5ubsta:nces regulated by this article as defined in Section 925.01(c)(46). No significant industrial user who commences discharge to the sewer system after adoption of this ordinance shall be permitted to introduce pollutants into the system unm the need for slug discharge control plans or procedures has been evaluated by the City. Each existing significant industrial Ulsef shall be evaluated by the City ror ~he need for slug discharge control plans or procedures at least once by Ju!y 1,2012.

(b) Certain Users wm be required to prepare Slug Discharge Prevention and Contingency Plans (SDPC) showing facilities and operating procedures to provide this protection. These plans shaH be submitted to the-City for review and approval. All existing Users reqUired having 2m SDPC p'ans shaH submit such a plan w!thin three !1"iOnt.hs after notification from the City and comp~ete implemeCttaHon with six: months. Review and approval of such plans and operating procedures shall not relieve the User from the responsibility to modify the User's facility as necessary to meet the

requirements of the article. SDPC wi!! be reviewed at ieast once during

each permit cyde.

(c) 	SDPC P~ans shall address at a minimum, the following:

(1) Description of discharge practices, including nomoutine batch discharges.

(2) Description of stored chemicals.

(3) Procedures for immediately notifying the POTV\! Plant Manager of any accidenta' or slug discharge, Such notification must also be given for any discharge which would violate any of the prohibited discharges in Section 925.20 of this ordooance.

(4) Procedures to prevent adverse impact from any accidental or slug discharge. Such procedures include, but are not limited to, inspection and maintenance of stallage areas, handling and transfer of materials, loading and unloading operations, control of plant site runoff, worker training, bUilding of containment structures or equipment, measures for containing toxic organic poHUltants (including solvents), and/or measures and equipment for emergency response.

(d) In the case of a siug discharge, ot is the responsibility of the User to immediately notefy the paTIN of the incident by telephone or in person. Information concerning the ~ocation of the discharge, type of waste, concentration and volume, and corrective action shall be provided by the User.

Within five days following a s~ug discharge, the User shalll subm!t a

detailed written report describi!1Jg the cause of the discharge arna1 the

measures being taken by the User to prevent similar futlnre occurrences.

Such notification shalt not reHeve the User of any expense, loss, damage,

or other liability which m8JY be incurred as a result of dameoge to the

sewerage system, fish kins, or any other damage to person or property, nor

shal! notification re~ieve ~he User of any fines, civil penalties, or other

liability which may be imposed by this ord!nance or other applicable law.

(e) A notice shall be permanently posted on the User's premises advising employees of a contact to cal! in the event of a s~ug discharge. The User shaH ensure that a!! employees who may cause or allow such slug dischalfge to occur are advised of the proper emergency notification procedure.

(f) 	Each User shall noU:ify the Plant filia;nager of any significant changes to the User's operations or system which might aiter the nature, quality, or volume of its wastewater at least 30 days prior to the change.

(Ord. 69-1991. Passed 4-15-91.)

925.32 WASTEWATER DJ8CHARGE PERMITS

(a) All industrial dischargers proposing to connect to or to discharge. industrial wastes and other wastes to the City's wastewater treatment plant shall obtain a Wastewater Discharge Permit before connecting to or discharging to the City's wastewater treatment plant. All existing industrial dischargers connected to or discharging to the City's wastewater treatment plant shall obtain a Wastewater Discharge Permit within ninety days after the effective date of this chapter.

(b) Industrial dischargers shall complete and file with the P~ant Manager, a permit application in the form prescribed by the Plant Manager and accompanied by the appropriate fee as required by Section 925.55. Existing industrial dischargers shall apply for a Wastewater Discharge Permit within thirty days after the effective date of this chapter and proposed new dischargers shall apply at least ninety days prior to connecting to the City's wastewater treatment plant. No discharge permit shall be issued unless and until disclosure is made by the discharger to the Plant Manager of all information required by the permit application including, but not limited to: descriptions of plant processes, rates of flow, 'A'aste'l/ater characteristics, and the point at which such 'vvaste...vater is or will be disoharged fnto City's sewerage system.

(1) 	Identifying Information.

A. 	The name and address of the facility, inch.Kiing the narr~e of the operator aJElid owner.

B. 	Contact information, description of activities, facilities, and pfant production processes on the premises.

(2) Environmental Permits. A list of any envirOU1lmentai control permits held by or for the facility.

(3) 	Description of Operations.

A 	A brief description of the nature, average rate of production (inducHng each product produced by type, amount, processes, and rate of produdnon). and standard industria! c!assifkations of the operation(s) carried out by such User. This description should include a schematk process diagram, which indicates poin1ts of discharge to the POTW from the regulated processes.

B. 	Types of wastes generat.ed, and a lost of all raw materials and chemicals used or stored at the facHity which are, or cou~d accidentally or ontentiol1ally be discharged to the POTW.

C. 	Number and type of employees, hours of operation, and proposed or actual hours of operation.

D. 	Type and amount of raw materials processed (average and! maximum per day}.

E. 	Snte p,a01s, floor plans, mechanical and plumbtng plans, and details to show all sewers, f!ool" drains, and appurtenal1ces by size, location, and elevatfion, and a~1 points of dlischarge.

(4) Time and duration of discharges.

(5) The location for monitoring ali wastes covered bit the permit.

(6) Flow Measurement. Information showing the measmed average daily and maximum daHy flow, in gallons per day, to the POTW from regulated process streams and other streams, as necessary, to allow use of the combined wastewater formula set out in Section 2.2C (40 CFR 403.6(e}).

{7} Measurement of Pol!~tants.

The categorical Pretue·atment Standards applicable to each re9Ju~a~ect process and allY new categorically regUlated processes for existing sources.

The resupts of sampuing and! anaEys&s identify~ng the nature and! concentration, and/or mass, where required by the Pretreatment Standard or by the Plant Manager~ of regulated poi8urants in the dischargE! from each regulated process.

c. 	~n:stantaneous, daily maxumUlm, and Icmg~term average concentrations, or mass~ \Jtfhere required, shall be reported.

a. 	The sample shall be representative of daHy operatoons and shall be aQ1a~yzed in accordance with procedures set out in Section

925.41 of this ordinance. VVhere the standard requires compliance with a BMP or po~lution prevenHon al~emaUve, the User shai! submit documentation as requnred by the Piant Manager or the applicable standards to determine comp~iance with the Pretreatment Standard.

e. 	Sampling must be performed ~n accordance with procedures set out in Section 925.41 of this ordinance.

(8) Any requests for a monitoring waiver (or a renewal of em approved

monitoring waiver} for a ponlutal1t neither present not expected to be present in the discharge based on section 925.39.

(c) All permit applications for new or modified permits shall be signed by a principal executive officer of the discharger, and a qualified engineer (State of Ohio licensed professional), and all renewal applications for existing permits shall be signed by a principal executive officer of the discharger.

(d) The Superintendent Plant Manager will evaluate the complete application and data furnished by the discharger and may require additional information. Within thirty days after full evaluation and acceptance of the data furnished, the Safety-Service Director shall issue a Wastewater Discharge Permit subject to terms and conditions provided herein.

(e) Compliance with categorical pretreatment standards when applicable is required for new sources upon promulgation. New sources are required to install and start up any necessary pollution equipment before beginning to discharge. Compliance with categorical standards when applicable is required within the shortest feasible time, not to exceed ninety days, after commencement of discharge.

(1) The term "New Source" means any building, structure, facility or installation from which there is or may be a discharge of pollutants, the constructIon of which commenced after the publication of proposed Pretreatment Standards under Section 307(c) of the Act which will be applicable to such source if such standards are thereafter promulgated in accordance with that section, provjded that:

A. The bUilding, structure, facllity or installation is constructed at a sJte at which no other source is located; or

B. The building structure, facility or installation totally replaces the process or production equipment that causes the discharge of pollutants at an existing source;

C. The production or wastewater generating process of the building, structure, facility or installation is substantially independent of an existing source at the same site. In determining whether these are substantially independent, factors such as the extent to which the new facility is engaged in the same general type of activity as the existing source should be considered.

(Ord. 69-1991. Passed 4-15-91.)

925,33 PRETREATMENT SCHEDUUNG REQUIREMENT

Where additional pretreatment and/or operation and maintenance activities will be required to comply with this chapter, the discharger shall provide a declaration of the shortest schedule by which the discharger will provide such additional pretreatment and/or implementation of additional operational and maintenance activities,

(a) The schedule shall contain milestone dates for the commencement and completion of major events leading to the construction and operation of additional pretreatment required for the discharger to comply with the requirements of this chapter including, but not limited to, dates relating to hiring an engineer, hiring other appropriate personnel, completing preliminary plans, completing final plans, executing contract for major components. commencing construction, completing construction, and all other acts necessary to achieve compliance with this chapter.

(b) Under no circumstance shall the P~ant Manager permIT a time increment for any single step directed toward compliance which exceeds nine months.

(c) Not later than fourteen days following each milestone date in the schedule and the final date for compliance, the discharger shall submit a progress report to the Plant Manager, including no less than a statement as to whether or not it complied with the increment of progress represented by the milestone date and, if not, the date on which it expects to comply with this increment of progress, the reason for delay, and the steps being taken by the discharger to return the construction to the approved schedule. In no event shall more than nine months elapse between such progress reports to the Plant Manager.

(Ord. 69-1991, Passed 4-15-91.)

925.34 PERMIT MODIFICATIONS

The Plant Manager may modify an individual Wastewater Discharge Permit for good cause, including, but not limi~ed to, the foHowing reasons:

(a) The Plant Manager reserves the right to amend any Wastewater Discharge Permit issued hereunder in order to assure compliance by the City with applicable laws and regulations. Within nine months of the promulgation of a National Categorical Pretreatment Standard, the Wastewater Discharge Permit of each discharger subject to such standards shall be revised to require compliance with such standards within the time frame prescribed by such standards. All National Categorical Pretreatment Standards adopted after the promulgation of this chapter shall be adopted by the City as a part of this chapter. Where a discharger, subject to a National Categorical Pretreatment Standard, has not previously submitted an application for a Wastewater Discharge Permit as required, the discharger shall apply for a Wastewater Discharge Permit from the Safety-Service Director within 180 days after the promulgation of the applicable National Categorical Pretreatment Standard by the U.S. Environmental Protection Agency. In addition, the discharger with an existing Wastewater Discharge Permit shall submit to the P~aJnt Manager within 180 days after the promulgation of an applicable National Categorical Pretreatment Standard, the disclosure information required, The discharger shall be informed of any proposed changes in his permit at least thirty days prior to the effective date of change. Any changes or new conditions in

the permit shall include a reasonable time schedule for compliance.

(b) The P~arnt Manager may change production based-categorical pretreatment standards to equivalent concentration limits to facilitate compliance evaluation. These equivalent limits shall be deemed Pretreatment Standards. Industrial Users will be required to comply with the equivalent limitations in lieu of the promulgated categorical standards from which the equivalent limits were derived.

(c) Any Industrial User operating under a control mechanism incorporating equivalent mass or concentration limits calculated from a production-based standard shall notify the Plant Manager within two business days after the Industrial User has a reasonable basis to know that the production level will significantly change within the next calendar month. Any User not notifying the Plant Manager of such anticipated change will be required to meet the mass or concentration limits in it control mechanism that were based on the original estimated of the long-term average production rate.

(d) Any Industrial User subject to equivalent mass or concentration limits estabUshed by the Manager must inc~ude a reason.ab~e meaSQ..!ire of the User's long"term production rate in its periodic compliance report.

(e) A change in the POTW that requires either a temporary or permanent reduction or elimination of the authodzed dfisclharge.

(f) Informatton indicating that the permitted d~sch8lrge poses a threat to the City's POTWl persorrmel, receiving waters, or affects the beneficial use of the plant's biosoinds.

(g) Vio!ation of any terms or conditions of tbe individual wastewater discharge perm~t.

(h) Misrepresentations or failure to fully disclose ail reievant facts in the wastewater discharge permit application or in any required reportung.

(i) Revision or a grant of variance from categorical Pretreatment Standards pursuant to 40 eFR 403.13

(j) To correct typographical or other errors in the indlivk:hJa! wastewater discharge permit.

(k) To reflect a transfer of the facility ownership or operation to a new owner or operator where requested in accordance with Section 925.37. (Ord. 69-1991. Passed 4-15-91.)

925.35 PERMIT CONDITIONS

Wastewater Discharge Permits shall specify no less than the following:

(a) Fees and charges to be paid upon initial permit issuance;

(b) Limits on the average and maximum wastewater constituents and eflaracteristies regulated thereby;

(c) Limits on average and maximum rate and time of discharge and/or requirements for flow regulations and equalization;

(d) Requirements for Installation and maintenance of inspeotion and sampling facilities;

(0) Special conditions that the Manager may reasonably require under particular ciroumstances of a given discharge including sampling locations, frequency of sampling, number, types and standards for tests and reporting schedule;

(f) Compliance schedules;

(g) Requirements for submission of special technioal reports or discharge reports where same differ from those prescribed by this chapter.

(h) (Ord. 69-1991. Passed 4-15-91.)

(a) A statement that indicates the wastewater discharge permit's issuance date, expiration date, and effective date.

(b) A statement that the wastewater discharge permit is nontransferable without prior rcotificat~o!1 to the Git~r in accordance with Section 925.37 of this ordinance, and provisions for furnishing the new owner or operator with a copy of the existing \fl.lastewater discharge permit.

(c) Effluent ~imits, including Best Management Practices, based on appJicabie Pretreatment Standards.

(01) Self monitoring, sampling, reportinQJ, notification, and record keeping requirements. These requirements shall include an identification of poiiutants (or best management practice) to be mOlllitored, sampUng location, sampling frequency, and sample type based on federall State,

and local law

(e) The process for seeking a waiver from monitoring for a pollutant neither present nor expected to be present in the discharge in accordance wi~h Section 925.34.

(f) 	A statement of applicable civil anal criminal penalties for violation of Pretreatment Standards and' Requirements, and any applicable compliance schea1u~e. Such schedule may not extend the time for compliance beyond that required by applicable federal, State, or local law.

(g) Requirements to control s~ug discharge as described in Section 925.30, if determined by the Piant Manager to be necessary.

(h) Any grant of the monitoring waiver in accordance with Section 925.34 by the Plant Manager must be included as a condition in the User's permit

925.37 LIMITATIONS ON PERMIT TRANSFER

Wastewater Discharge Permits are issued to a specific discharger for a specific operatien and are not assignable to anether discharger withB-ttt the pr~ approval of the Safety Service Director, or transferable to any other location. (Ord. 69-1991. Passed 4-15-91.)

Wa:stowater Discharge Permits may be transferred to a new owner or operator only if the permittee gives at least 90 days advance notice to the Plarllt Manager and the Plant Manager approves the individual wastewater discharge permit transfer. The notice to the Plant Manager must indude a written certification by the new owner or operator which:

(a) States that the new owner and/or operator has flO immediate intent to change the facility's operatnons or processes

(b) Identifies the specific date on which the transfer is to occur.

(c) Acknowledges full responsibility for complying with the existing

individua! wastewate·r discharge permit. Failure to provide advance notice of a transfer renders the individual wastewater discharge permit void as of the date of the facDlity transfer.

· ~.. " 'I

925.38 GOMPLI/\NCE DATE REPORT IN[TlAL COMPLIANCE REPORT

Within 90 days following the date for final compliance with applicable categorical

Pretreatment Standards or in the case of a New Source following commencement of the

introduction of wastewater into the POTW, any Industrial User subject to Pretreatment

Standards and Requirements shall submit to the City a report containing the information described jn Section 925.60. For Industrial Users subject to equivalent mass or concentration limits established by the City in accordance with Section 925.34, this report shall contain a reasonable measure of the User's long-term production rate. For all other Industrial Users subject to categorical Pretreatment Standards expressed in terms of allowable pollutant discharge per unit of production (or other measure of operation), this report shall include the Industrial User's actual production during the appropriate sampling period. (Ord. 69-1991. Passed 4-15-91.)

925.39 PERIODIC COMPLIANCE REPORTS AND MON~TORiNG WAfVERS

(a) Any discharger subject to a Pretreatment Standard set forth in this chapter, after the compliance date of such Pretreatment Standard, or, in the case of a new discharger, after commencement of the discharge to the City's wastewater treatment facilities, shall submit to the Ptant Manager during the months of June and December, unless required more frequently in the pretreatment standard or by the City, a report indicating the nature and concentration of prohibited or regulated substances in the effluent which are Ilmited by the Pretreatment Standards hereof. In addition, this report shall include a record of all measured or estimated average and maximum daily flows during the reporting period reported hereof. Flows shall be reported on the basis of actual measurement, provided however, where cost or feasibility considerations justify, the Plant Manager may accept reports of average and maximum flows estimated by verifiable techniques. The Plant Manager, for good cause shown considering such factors as local high or low flow rates, holidays, budget cycles, or other extenuating factors may authodze the submission of such reports on months other than those specified above.

(b) .In cases where the Pretreatment Standard requires compliance with a Best Management Practice (BMP) or poHution prevention altemaUv19, the ~ndustrlal User must submit documentation required by the Plant Manager for the Pretreatment Standard necessary to determine compliance statUls of the industria! User.

(c) Reports of permittees shall contain all results of sampling and analysis of the discharge, inclUding the flow and the nature and concentration, or production and mass where required by the Plant Manager. This inciuoles monitoring of any poHUltant more frequently than required by the City. The frequency of monitoring by the discharger shall be as prescribed by the City. All analyses shan be performed in accordance with 40 CFR, Part 136 and amendments thereto.

(d) The PI<1int Manager shall require appropriate reporting from those Industrial Users with discharges that are not subject to categorical Pretreatment Standards.

(e) Reports must be based on an appropriate amount of sampling and analysis performed during the period covered by the report. Each biannual report shall contain at least some data for the period covered by the report.

(f) All monitoring which is performed by the lndustdaf User using these procedures approved under 40 CFR Part 136 or approved alternatives shall be reported in periodic compliance reports.

(g) If sampling performed by an Industrial User indicates a violation, the User shall notify the City within twenty-four hours of becoming aware of the violation. The User shall also repeat the sampling and analysis and submit the results of the repeat analysis to the City within thirty days after becoming aware of the violation, except the Industrial User is not reqUired to resample if:

(1) The City performs sampling at the Industrial User at a frequency of at least once per month.

(2) The City performs sampling at the User between the time when the User performs its initial sampling and the time when the User receives the results of this sampling.

(h) Ail wastewater samples must be representative of the User's dnscharge. Wastewater monHoring and flow measurement facilities shall be properly operated, kept clean, and maintained in good working order at alII times. The failure of a User to keep its monitoring facility in good working order shall not be grounds for the User to claim that sample results are unrepresentative of its discharge.

(i) Monitoring Waivers -The City, at its discretion, may authorize an Industrial User subject to a categorical Pretreatment Standard, except for centralized waste treatment facilities regulated by and defined in 40 CFR 437, to forego sampling of a poUutant regulated by a categorical Pretreatment Standard ifthe Industrial User has demonstrated through sampling! and other technicall factors that the pollutant is neither present nor expected to be present in the discharge, or is present only at backgmund levels from intake water and without any increase in the poUutant due to activities of the industria! User. [See 40 CFR 403.12(e)(2)] This authorization is subject to the following conditions:

(1) The waiver malY be authorized where ell pollutant is determined to be present sanely due to sanitary wastewater discharged ¥rom the facility providled that the sanitary wasftewater is not regulated by an applicable caJtegorical standalld and otherwise ~nciudes no process wastewater.

(2) The monitoring waiver is valid only for the duration of the effective period of the individual wastewater discharge permit, but in no case longer than five yeZlrs. The User must submit a new request for the waeveu beiore the waiver can be granted for each subsequent individual wasteWa1ter discharge permit. See Section 925.32.

(3) In making its request for a poUutant monitoring exemption, the Industrial User must demonstrate through sampling and other technical factors that the pollutant is neither present nor expected to be present in the discharge, or is present on~y at background levels from intake water and! wi~hout any increase in the pollutant due to activities of the Industri611 User. In making a demonstration that a pollutant is not present, the industrial User must provide data from at least one samp!ing of the facility's process wastewater prior to any trreatmerllt present at the facility that is representative waJstewater from aH processes. Non-detectable sample results may only be used as a demonstration that a pollutant is not present if the USEPA approved anafiytical method from 40 CFR: 136 with the lowest method detection limit for that pollutant is used.

(4) The request for a monitoring waiver shaH be s~gned by an authorized representative of the lr11chJlstria! User in accordance with paragraph (F) of rule 3745-3-06 of the Administrative Code, and it must include the certifica~ion statement in 40 CFR 403.6 (a)(2)(ii}.

(5) Non-detectable sample results may be used on~y as a demonstration that a pollutant is !lot present if an EPA approved method from 40 CFR Part 136 w1th a detection leve! that is be~ow the fimet.

(6) Ptny grant of the monitoring waive-I" by the Plant Manager rnus~ be irnciuded as a condition in the User's permit The reasons supporting

the waiver and any information submitted by the Userr in its request for the waiver must be maintained by the Plant Manager for three years after expiration of the waiver.

(7) Upon approval of Hve monitoring waiver and revision of the 1U's c08l1ho! mechanism by the contro! authority, the ~U shalt certify on each salfA monitoring report with the following statement: "BaseD' on my inquiry of the person orpersons directly responsible for managing compliance with the pretreatment standards under 40 CFR (specify applicable national pretreatment stano'ard part orparts), I certify that, to the best of my knowledge and belief, there has been no increase in the level of(list pollutant or pollutants) in the wastewaters due to the activities at the facility since submittal of the fast periodic report under paragraph (E) of rule 3745~3~06 of the Administrati\re Code."

(8) In the event that a waived pollutant is found to be present or is expected to be present because of changes that occur in the IU's operations, the IV must immediately: Notify the control authority in writing, begin monitoring that poHutant 8lnd comply with the monitoring requirements of Section 925.39(a}, or other more frequent monitoring requirements imposed by the Plant Manager, and notify the P~ant Manager.

(9) This provlsDon does not supersede certCficatoon processes and requirements established in categorical Pretreatment Standards, except as otherwise specified in the categorical Pretreatment Standard.

('10) 	Monitoring waivers do not apply to pollutants where there are certification processes and requirements estabHshed by the control authority or by categorical! pretreatment standards (e.g. TOMP/certification alternative to total toxic organics monitoring) unless allowed fOIr by the appiicabie categorical pretreatment standards

(11) 	The monitoring waiver applies ordy to ~U seif·monutoring and does not remove the POTI'V's obfigations for IU monitoring for that parameter. However, the waiver can be extended to POTW monitoring done to satisfy ~U selfwmonitoring.

(Ord. 69-1991. Passed 4-15-91.)

925.40 MONITORING FACILITIES

(a) All Users who propose to discharge or who in the judgment of the City could now or in the future discharge wastewater with constituents and characteristics different from that produced by a domestic premise may be required to install a monitoring facility.

(b) Monitoring Facility Requirements

(1) Each discharger shall provide and operate at the discharger's own expense, a monitoring facility to allow inspection, sampling, and flow measurement of each sewer discharge to the City's wastewater treatment facilities. Each monitoring facility shall be situated on the discharger's premises, except where such a location would be impractical or cause undue hardship on the discharger, the Manager may concur with the facility being constructed in the public street or sidewalk area providing that the facility is located so that it will not be obstructed by landscaping or parked vehicles,

(2) There shall be ample room in or near such sampling facility to allow accurate sampling and preparation of samples for analysis. The facility, sampling, and measuring equipment shall be maintained at all times in a safe and proper operating condition at the expense of the discharger.

(3) All monitoring facilities shall be constructed and maintained in accordance with all applicable local construction standards and specifications. Construction shall be completed within 120 days of receipt of the City's authorization to proceed.

(Ord.69-1991. Passed 4-15-91.)

925.41 INSPECTION AND SAMPLING

The Plant Manager or his authorized agents may inspect the monitoring facilities of any discharger to determine compliance with the requirements of this chapter. The discharger shall allow the Plant Manager or his authorized agents, upon presentation of credentials of identification to enter upon the premises of the discharger at all reasonable hours, for the purposes of inspection, sampling or records examination. The Manager shall have the right to set up on the discharger's property such devices as are necessary to conduct sampling, inspection, compliance monitoring and/or metering operations.

Sample Analysis

(a) All collected samp~es must be of such nature that they provide a true and accurate representation of the industry's norma! workday effluent quatuty.

(b) ChaDn-of~custody procedures, sample preservation technuques, and! samp~e holding times recommended by EPA shan be followed in aU self~monitoring activities.

(c) Monitoring shail be periormed at the approved monitoring staltion on the effluent sewer. location and design of the monitoring station shall be subject to the revuew and approval of the City Engineer. Any change tn monitoring location will be subject to the alpproval of the City Engineer.

(d) All analyses sha~1 be performed in alccordance Wit!1 procedures estabHshed by EPA under the provisions of Section 304(h) of the Act [33 USC 1314(h)] and contained in 40 CFR, Part 136, and amendments thereto or with any other test procedures approved by EPA or the City. Sampling shaDI be pertormed in accordance with the techniques approved by EPA or the Cay.

(e) Except as indicated in Subparagraphs (f) and (9) below, the User must collect wastewater samples using flow proportional composite sampling techniques, unless time~proportionalcomposite sampHn9 or grab samp~~ng is authorized by the Plant Manager. Where time~proport8cmalcomposite sampling or grab sampling fS authorized, the samples must be representative of the discharge. Using protocols (includling Cl!ppmpria~e preservation) specified in 40 CFR Part 136 and appropriate EPA guidance, multiple grab samples collected during a 24~hour period may be compostted prior to analysis as foilows: for hexavalent chronlGum, cyanide, total phenols, and sulfides, the samples may be composited in a laboratory or in the field; for volatile organics and oil and grease, the samples may be composited in the laboratory. Composite sampies for other parameterrs unaffected by the compositing procedures as

documented in approved EPA methodologies may be authorized by the

Plant Manger, as appropriate. Sn addition, grab samples may be required to

show compliance with instantaneous limits.

(f) 	Samples for oil and grease, temperature, pH, cyanide, total phenols, sulfides, and volatile organic compounds must be obtained using grab collection techniques. Alternately, pH compliance may be assessed through the use of a strip~chart Of a circular chart over the monitoring period from a continuous pH recordJerj at the discretion of the Plant Manger.

(g) For sampling required in support of baselDne monitoring and initial compliance reports required in Section 925.38 and 40 CFR 503.12 (b) and {ol}, a minimum of four grab samples must be used for pH, c~taniole, total phenols, oil and grease, sulfides) and volatiie organic compounds for facilities for which historical sampHng data do not exist. For facilities for which historical sampling data are available, the Plant Manager may authorize a lower minimum. For reports required by Section 925.38 and 40 CFR 403.12 (e) and (h), the Indus~rial User is required to collect the number of grab samples necessary to assess and assure compiiance with applicable Pretreatment Standards.

(h) ~f a violation is detected through sampiing and analys~s conducted by the City in lieu of the lndustrial User, the City shall perform the repeat sampling and analysis within 30 days of becoming aware of the violation lm~ess it notifies the User of the violation and requkes the User to perform the repeat sampuing and analysis.

(Ord. 69-1991. Passed 4-15-91.)

925.42 CONFIDENTIAL INFORMATION

Information and data furnished to the Safety-Service Director or Plant Manager with respect to the nature and frequency of discharge shall be available to the public or other governmental agency without restriction unless the discharger specifically requests and is able to demonstrate to the satisfaction of the Safety-Service Director or Plant Manager that the release of such information would divulge information, processes or methods of production entitled to protection as trade secrets or proprietary information of the discharger.

(a) When requested by a discharger furnishing a report, the portions of a report which may disclose trade secrets or secret processes shall not be made available for inspection by the pUblic but shall be made available upon written request to governmental agencies for uses related to this chapter, the NPDES Permit, State Disposal System pennit and/or the Pretreatment Programs; provided, however, that such portions of a report shall be available for use by the State or any State agency in judicial review or enforcement proceedings involving the discharger furnishing the report. Wastewater constituents and characteristics will not be recognized as confidential information.

(b) Information accepted by the Safety-Service Director or Plant Manager as confidential shall not be transmitted to any governmental agency or to the general public by the Safety-Service Director or Plant Manager until and unless ten-day notification is given to the discharger.

(Ord.69-1991. Passed 4-15-91.)

925.43 EMERGENCY SUSPENSJON OF SERVICE AND DISCHARGE PERMITS

The Plant Manager may for good cause shown suspend the wastewater treatment service and the Wastewater Discharge Permit of a discharger when it appears to the Plant Manager that an actual or threatened discharge presents or threatens an imminent or substantial danger to the health or welfare of persons, substantial danger to the environment, interfere with the operation of the City's wastewater treatment plant, violate any pretreatment limits imposed by this chapter or any Wastewater Discharge Permit issued pursuant to this chapter. Any discharger notified of the suspension of the City's wastewater treatment service and/or the Discharger's Wastewater Discharge Permit, shall within a reasonable period of time, as determined by the Plant Manager, cease all discharges. In the event of failure of the discharger to comply voluntarily with the suspension order within the specified time, the Plant Manager shall commence judicial proceedings immediately thereafter to compel the discharger's compliance with such order. The Plant Manager shall reinstate the Wastewater Discharge Permit and/or the wastewater treatment service and terminate judicial proceedings pending proof by the discharger of the elimination of the noncomplying discharge or conditions creating the threat of imminent or substantial danger as set forth above. (Ord. 69-1991. Passed 4-15-91.)

925.44 REVOCATION OF PERMIT

The Plant Manager may revoke the permit of any discharger which fails to:

(a) Factually report the wastewater constituents and characteristics of its discharge;

(b) Report significant changes in wastewater constituents or characteristics;

(c) Refuses reasonable access to the discharger's premises by representatives of the Manager for the purpose of inspection or monitoring; or

(d) Violates the conditions of its permit, or this chapter, or any final judicial order

entered with respect thereto. (Ord.69-1991. Passed 4-15-91.)

925.45 NOTIFICATION OF VIOLATION; ADMINSTRATIVE ADJUSTMENT

Whenever the Plant Manager finds that any discharger has engaged in conduct which justifies revocation of a Wastewater Discharge Permit. the Plant Manager shall serve or cause to be served upon such discharger a written notice by personal service or by certified mail with return receipt requested, stating the nature of the alleged violation. Within 30 days of the date of receipt of the notice, the discharger shall respond personally or in writing to the Plant Manager, advising of its position with respect to the allegations. Thereafter, the parties shall meet to ascertain the veracity of the allegations, and where necessary, establish a plan for the satisfactory correction thereof. (Ord.69-1991. Passed 4-15-91.)

925.46 SHOW CAUSE HEARING

Where the violation of Section 925.44 hereof is not corrected by timely compliance by means of administrative adjustment, the Plant Manager may order any discharger which causes or allows conduct prohibited by Section 925.44 hereof, to show cause before the Plant Manager or its duly authorized representative, why the proposed permit revocation action should not be taken. A written notice shall be served on the discharger by personal service or by certified mail with return receipt requested, specifying the time and place of a hearing to be held by the Plant Manager or his designee regarding the violation, the reasons why the enforcement action is to be taken, the proposed enforcement action, and directing the dischargerto show cause before the Plant Manager or its designee why the proposed enforcement action should not be taken. The notice of the hearing shall be served no less than ten days before the hearing. Service may be made on any agent, officer, or authorized representative of a discharger. The proceedings at the hearing shall be considered by the Plant Manager which shall then enter appropriate orders wrth respect to the alleged improper activities of the discharger.

Appeal of such order may be taken by the discharger in accordance with applicable local or State law. (Ord.69-1991. Passed 4-15-91.)

925.47 JUDICIAL PROCEEDINGS

Following the entry of any order by the Plant Manager with respect to the conduct of a discharger contrary to the provisions of Section 925.44 the attorney for the Plant Manager may, following the authorization of such action by the Plant Manager, commence an action for appropriate legal and/or equitable relief in the appropriate local court.

(Ord.69-1991. Passed 4-15-91.)

925.48 PRETREATMENT VIOLATION ENFORCEMENT RESPONSE POLICY

(a) The following definitions and response criteria shall be used in responding to violations of the City of Massillon Industrial Wastewater Pretreamtent Program.

(b) Definitions

(1) Major Violation. Major violations are those that exceed the limits frequently and/or by a large quantity (e.g. the technical review criteria under Paragraph 6), impede the determination of compliance status, have the potential to cause or may have actually caused adverse environmental effects, health problems, or interfered with the POTW treatment capability. Any vilation that meets the definition of Significant Noncompliance (Paragraph 6) is considered a major violation.

(2) Significant Noncompliance (SNC). Any violation of pretreatment requirements (limits, samplings, analysis, reporting, and meeting compliance schedules, and regulatory deadlines) is an instance of noncompliance for which the Industrial User is liable for enformcement, including penalties. Instances of SNC are Industrial User violations which meet one or more of the following critieria:

(c) Violations of Wastewater Discharge Limits

(1) Chronic violations of wastewater discharge limits, defined here as those in which sixty-six percent (66%) or more of all of the measurements taken for the same pollutant during a six-month period exceed (by any magnitude) the daily maximum limit or the average limit for the same pollutant paramoter;a numeric Pretreatment Standard or ReqUirement, including ~nstaUltaneous limits as defined in Section

925.01;

(2) Technical Review Criteria (TRC) violations, defined here as those in which thirty-three (33%) percent or more of all of the measurements for each pollutant parameter taken during a six-month period equal or exceed the product of the daily maximum limit or the average limit a numeric Pretreatment Standard or Requirement, including Instantaneous Limits as defined by Sedion 925.01 multiplied by the applicable TRC (TRC= 1.4 for BOD, TSS, fats oil and grease, and 1.2 for all other pollutants except pH);

(3) Any other violation of a pretreatment effluent limit (daily maximum. Sf longer term average long term average, in:s~antaneoUis Limit, or narrative standard) that the Safety-Service Director or Plant Manager determines has caused. alone or in combination with other discharges, interference or pass through (including endangering the health of POTW personnel or the general public);

(4) Any discharge of a pollutant that has caused imminent endangerment of human health, welfare or to the environment or has resulted in the POTW's exercise of emergency authority to halt or prevent such a discharge;

(d) Violations of Compliance Schedule Milestones. Vio~81tiQns of compliance schedule mHestones that are contained in a local control mechanism or enforcement order, for starting construction, completing construction, and attaining final compliance by 90 days or more after the schedule date.

(e) Failure to Provide Reports. FaHure to provide reports for compliance scheduled, self-monitoring data, or categorical standards (baseline monitoring reports, 90 day compliance reports, and periodic reports, reports on compliance with categorical Pretreatment Standard deadlines, periodlic self~monitoring reports, and reports 001 compliance with compliance schedules) within 30 days from the due date.

(f) 	Failure to accurately report noncompliance.

(g) Any other violation or group of violations, which may include a violation of Best Management Practices, that the City considers to be significant by adversely affecting the operation or implementation of the local pretreatment program.

(h) 	Industrial User (IU) Violation Response Categories Level 1 (L1) " Telephone Call Level 2 (L2) -Notice of Violation (NOV) Level 3 (L3) ~ Administrative Order (AO) Level 4 (L4) -Administrative Order and/of Penalty Level 5 (L5) -Administrative Order and/or Penalty and/or Judicial Action Level 6 (L6) -Administrative Order and/or Penalty and/or Judicial Action and/or

Show Cause Hearing and/or Termination of Service

(i) 	Titles and Timeframes for Industrial Users Violations Response Categories

Level 1 -Immediately, Pretreatment Coordinator, Pretreatment Inspector

Level 2 -5 Days, Pretreatment Coordinator, Pretreatment Inspector, Plant

Manager

Level 3 -10 Days, Plant Manager, City Engineer, Safety-Service Director

Level 4 -10 Days, Plant Manager, City Engineef, Safety-Service Director

Level 5 -30 Days, Plant Manager, City Engineer, Safety-Service Director, Law Director

Level 6 -30 Days, Plant Manager, City Engineer, Safety-Service Director, Law Director

G) 	Pretreatment Enforcement Criteria

The follOWing pages represent the City's Pretreatment Enforcement Response Criteria:

| NONCOMPLIANCE | NATURE OF VIOLATJON | RANGE or RESPONSES |  |
| --- | --- | --- | --- |
| A. lLLEGAL DlSCHARGE |  |  |  |
| I . NOllpennillcd dischargc | F~jjllrc 10 ilpply I'lli' pcnnil rencwal, 110 cnvirolllUcnu,l (ll POTW damagc | IA |  |
| 2, UnrcllllillCd disclmrgc | Di~chargcT ull:,w1111l of pcrillil rcquirelllclIl. no cnvirontt:nl,d or POTW <l;J1ll~gc | D |  |
| 3. Unpenllille<) discharge | RCSlIll' in viol<tliun (ll'l'O'l'W NPDES penn;l, ur cillngcrous si!ullliOJJ -(SNC) | L5 |  |

NONCOMPUANCE

NATURE OF VJOLATION

RANGE Of RESPONSGS

B. DlSCHA1WE STANDARD VIOLATION

lsolated. nonsignificant

LI . Initial \·jollllion

I. Excecdi.U1ce of d;schnrgc lim its (locn) or

« 120'1.. of limit)

cUlcgnl'iclll)

L1 -Subsequenl violalions

2. Excceclallcc of di~cl1argc limits

Frequcnt, nonsigniflcanl

IA

(rcreated of(ensc)

J. Excecdancc o( discharge lillJ;IS

SNC

L5

Results in known cnvjronilleillal r,,' POTW

4. Excccd:lllce of discharge limits

L5

onnmgc (cndmlgcrmcm 10 life) 5, Slug load discharge

IsolulCd witholll known damage

L3

6. Slug load disch~rge

Isolated \ViOl known intcrference. Jla~s

L5

throu¥h, or d~nlnge resuhs -(SNC)

Recurring -(SNC)

7. Slug load discharge

1..6

C. SAMPLlNG, MONITORING AND REPORTING VIOLATIONS I, Minor sampling, moniloring 01' reponing

Isol~led or infrequent

Ll -lnitia I violal ion

(1" or 2'" offense)

L2 -Subsequent viol.linn

deflciencie.~

Fre'lucm (repenled offcnse) (II' COlllinuous

L3

2. Minor smnplinS I monitoring or reporting

deficiencies

Isolaled or infrequent

L3

3. Major sampling, mOlliloring or reponing

(I" or 2'" ofCense) 4, Major s81l1pl ing, man iloring or reponing

deficiencies

Frequcnt (reflCl1(cd o((ense) or COl1liouous 10

IA

becomeSNC

dcCicicncies

SNC

1.5

5. Complele failure to s~l11ple, monitor or

reron

Violalion or NOY contlilioll

L6

6. Con(inued f"ilure lO snmrlc. "'onilO< or

repOli 7, Failure to submit schedule of

Violation of consenl order or AO

L6

eOlnplimlee (SOC)

ISOla led or inCrequenl No known effcels

L2

8. Failure 10 noliry of erouenl limit

violation oC slug discharge 9, Fuilurc tn nOliCy oC cmuCOI lionit

FJ'e'J0ent 01' eominued \'inlaliull -(SNC)

L5

violation o( slug discharge

Known cn\,irolllllcJltnl or rOTW damage

LG

10. Failul'C 10 oOlify oC emu~J1I li'll;1

resuhs • (SNC)

violation or slu~ discharge

RANGE OF RESPONSES

NATURE OF VIOLATION

NONCOMPLIANCE

D. COM I'LlANCE SCHEDULE

WillllUl affcct mile! mllcmme dalc~ or nllal

L2

I. Mi,,,eu lIIilcSlulll' dale

datc

Wlfl "ffeel other luill'Sl"n" llT lin,,1 dalc.

LJ

2. Missed lllUCS(Olle {\;lIe

ViolOlion for good or val id cause

WillllfJ'cCl Olher mileslOlie or fillal (hllc.

L5

3. Missed milestone dale

Vllll;lti"ll llOl for a good Cll' vulid COliSC (SNC)

Did not sHllmi! report bUi dill complele

L2 reponill~ requiremellts

4. Failure 10 meet compliallce schedule

mileslonc

Did nOI suhllliJ reporl alld diullol complele

L4

5. Failure IlImeCI c(lJHpliullce schedule

mileslonc

reponing rC(juirCUleJllS

Good or v<l1 id l·dU~C

12

6. Misscd fiJ],1! date

30 days OJ' more OUls14nding Failure or

L5 refusal 10 wlJlply wilhou! good or valid cause

7. Misscd IIlml dillc

Aoy insrallee . (SNC)

L5

Reponing false illfonnmiOlJ

SPILL INCIDENTS

Rcported <100 il1\'csligalcd

L3

j. Spill inc ide III

Failure lO upgrade or develop spill

L4

2. Repented spill incidcnt~

preventiOIl program

Failure to Rei on n decision of compli~ncc

L5

3. Re[1Caled spill incidenL~

mccting and results in known cnvironmcntal damug e or WWTP d~",age

F. VIOLATIONS DETECTED DURlNG FlELD INSPECTIONSIINVESTIGAT1ONS

Any in-l(,1l1cc

L2

J. Minor violnlion ofallalyttcal procedures

No cvidellce of iment

L3

2. Ml~or "iolotion Qf anal)'lical procedures

Evidence of negligence or intent (SNC)

1.5

3. Mnjor viololion of analylical procedures

No evidence of negligence or imenl

L3

~. Minor "iolatioll of pennil condition

Evi.dcoce uf negligence or intent

LA

5. Minor viol.,tio,) or penni! condition

6. Mlijur violalion of penni! condilion

Evidcncc of negligence or inlenl (SNC)

U

ENFORCEMENT

(a) Administrative Orders. The City has the authority to issue enforceable Administrative Orders to any or a[l dischargers to the sanitary sewer system. Such Orders may contain numerical discharge values limiting the volume, concentration or mass loadings from dischargers.

(b) Compliance Schedule. The City may issue Compliance Schedules to Dischargers who fail to achieve compliance with Administrative Orders issued by the City or numerical limitations of the City or other regulatory agencies.

(c) Litigation. Whoever violates an Order of the City or fails to comply with any

provisions of this chapter shall be guilty of a misdemeanor and shall be penalized in the manner set forth.

(d) Enforcement Actions: Annual PUblications. At least annually, the Safety-Service Director shall pUblish a list of all industrial users provide meaningful public notification in a newspaper of general circulation within the jurisdiction served by the P01Vl! of all industrial users which at any time during the previous twelve months were in significant noncompliance with applicable pretreatment requirements standards or other pretreatment requirements. For the purposes of this provision, an industrial user is in significant noncompliance if its violations meet one or more of the following criteria:

(5) Chronic violations of wastewater discharge limits, defined here as those in which sixty-six percent (66%) or more of all of the measurements taken for the same pollutant during a six-month period exceed (by any magnitude) the daily maximum limit or the average limit for the same pollutant parameter; a numeric Pretreatment Standard or Requirement, unciuding Instantaneous Limits as defined in Section

925.01;

(6) Technical Review Criteria (TRC) violations, defined here as those in which thIrty-three (33%) percent or more of all of the measurements for each pollutant parameter taken during a six-month period equal or exceed the product of the daily maximum limit or the average limit aJ numeric Pretreatment Standard or Requirement, including InstantaneoUls Limits as defined by Section 925.01 multiplied by the applicable TRC (TRC= 1.4 for BOD, TSS, fats oil and grease, and 1.2 for all other pollutants except pH);

(7) Any other violation of a pretreatment effluent limit (daily maximum. 9f longer term average long term average, instantaneous Limit, or narrative standard) that the Safety-Service Director or Plant Manager determines has caused, alone or in combination with other discharges, interference or pass through (including endangering the health of POTW personnel or the general public);

(8) Any discharge of a pollutant that has caused imminent endangerment of human health, welfare or to the environment or has resulted in the POTW1s exercise of emergency authority to halt or prevent such a discharge;

(9) Failure to meet, within Vio~ation by ninety days or more after the scheduled date of a compliance schedule milestone contained in a wastewater discharge permit or enforcement order for starting construction, completing construction, or attaining finall compliance;

(10) Failure to provide, 'Nithin thirty Elays after the due date, required reports such as baseline monitoring reports, ninety day compliance reports, periodic selfmonitoring reports, and reports on compliance with compliance schedules within 45 days of the due date;

(11) Failure to accurately report noncompliance;

(12) Any other violation or group of violations, which may include a violation of Best Management Practices, which the Safety~Service Director or Piant Manager determines will or has adversely affected the operation or implementation of the City's pretreatment program.

(Ord.69-1991. Passed 4-15-91.)

925.49 RIGHT OF APPEAL

Any discharger or any interested party shall have the right to request in writing an interpretation or ruling by the Plant Manager on any matter covered by this chapter and shall be entitled to a prompt written reply. In the event that such inquiry is by a discharger and deals with matters of performance or compliance with this chapter or deals with a Wastewater Discharge Permit issued pursuant hereto for which enforcement activity relating to an alleged violation is the subject, receipt of a discharger's request shall stay all enforcement proceedings pending receipt of the aforesaid written reply. Appeal of any final judicial order entered pursuant to this chapter may be taken in accordance with local and State law. (Ord.69-1991. Passed 4-15-91.)

925.50 OPERATING UPSETS

(a) Any discharger that experiences an upset in operations which places the discharger in a temporary state of noncompliance with this chapter or a Wastewater Discharge Permit issued pursuant hereto shall inform the Plant Manager thereof within twenty-four hours of first awareness of the commencement of the upset. Where such information is given orally, a written follow-up report thereof shaH be filed by the discharger with the Plaillt Manager within five days. The report shall specify:

(1) Description of the upset, the cause thereof, and the upset's impact on a discharger's compliance status.

(2) Duration of noncompliance, including exact dates and times of noncompliance, and if the noncompliance continues, the time by which compliance is reasonably expected to occur.

(3) All steps taken or to be taken to reduce, eliminate and prevent recurrence of such an upset or other conditions of noncompliance.

(b) A documented and verified bonafide operating upset shall be an affirmative defense to any enforcement action brought by the Psant: Manager against a discharger for any noncompliance with the chapter or any Wastewater Discharge Permit issued pursuant hereto, which arises out of violations alleged to have occurred during the period of the upset.

(Ord. 69-1991. Passed 4-15-91.)

925.51 ADMINISTRATIVE PENALTIES

Any discharger who is found to have violated an order of the Plant Manager, or who has failed to comply with any provision of this chapter and the regulations or rules of the Safety~Service Director, shall be penalized as follows:

(1) Continuous dischargers. The City may assess administrative penalties ranging in the amount of one hundred dollars ($100.00) to one thousand dollars ($1,000) per violation per day upon sewer users who normally have a continuous discharge who fail to comply with numerical values or administrative orders issued by the City or other regulatory agencies. Such administrative penalties shall be determined by the City based on the severity of the violation and the enforcement category assigned to the violator. If the violation is an "average" violation, the penalty shall be applied times the number of days (i.e., $100 x 30 = $3,000). If a discharger has both monthly average maximum and one day maximum violations for the same parameter, the monthly violation takes precedent. If the monthly average maximum and one day maximum violations are for different parameters, they will be assessed penalties as separate violations.

(2) Batch dischargers. The City may assess administrative penalties ranging in the amount of five hundred dollars ($500.00) to three thousand dollars ($3,000) per violation upon sewer users who batch discharge and fail to comply with numerical values or administrative orders issued by the City or other regulatory agencies. Such administrative penalties shall be determined by the City based on the volume of the discharge, the severity of the violation, and the enforcement category assigned to the violator. If the violations are for different parameters, they will be assessed penalties as separate violations.

NOTE: In addition to administrative penalties stated in Section 925.51 (a) "surcharges" may be imposed upon violating dischargers. Such surcharges are intended to recover the additional cost associated with treating "extra strength" discharges and are not penalties.

(a) State and Federal Law. In addition to Administrative penalties, dischargers that interfere with the proper operation or cause damage to the municipal wastewater system may be liable to State and/or Federal Law which provides for penalties up to one hundred thousand dollars ($100,000) per day and six years in jail for a repeat knowing criminal violation.

(b) Municipal Civil and Criminal Penalties. In addition to Administrative penalties, discharges which interfere with the proper operation of or cause damage to the municipal wastewater system may be liable to the City as follows:

(1) The City may seek Civil Penalties in the amount of three thousand dollars ($3,000) per violation per day; or

(2) The City may seek criminal penalties. Any person who is found guilty shall be guilty of a misdemeanor, and upon conviction thereof, shall be fined in an amount not exceeding one thousand dollars ($1,000) per day per violation. (Ord. 69-1991. Passed 4-15-91.)

Administrative Penalty Schedule

Any Industrial User violating any of the following regulations will receive administrative penalties accordingly.

| Discharge slug load which causes WWTP (POTW) upset |  | $ 500 |  |
| --- | --- | --- | --- |
| Discharge slug lO<ld which causes pass-through (Either of the above would also be subject to pay Massillon's NPDES fines for the cxcursion(s) |  | $1,000 |  |
| Deliberately discharge cOllstituents known to be in violation of local or categorical limits |  | $1,000 |  |
| Report or administrative penalty submitted late: |  |  |  |
| 1 [0 10 days |  | $ 250 |  |
| 11 to 20 days |  | $ 500 |  |
| 21 to 30 days | $ 750 |  |  |
| > 30 days |  | $1,000 |  |
| Failure to report (within 24 hours): |  |  |  |
| A change in discharge |  | $ 100 |  |
| Any pem1il discharge violation |  | $ 250 |  |
| A slug load (no harm to POTW) | $ 250 |  |  |
| A slug load (harm to POTW) |  | $ 500 |  |
| Failure to report: |  |  |  |
| Analysis for required parameters (per parameter) |  | $ 100 |  |
| Incomplete or missing files (recurrent) | $ 100 |  |  |
| Additional monitoriug (EPA approved method) |  | $ 250 |  |
| Reporting violations: |  |  |  |
| Report is improperly completed |  | $ 100 |  |
| Report is improperly signed or certified | $ 100 |  |  |
| Report is improperly signed or certified after notice by POTW |  | $ 250 |  |

| Unpermitted discharge: |  |  |  |
| --- | --- | --- | --- |
| UOflware of requirement (no harm) |  | NOV |  |
| Unaware of requirement (harm to POTW) | $ 500 |  |  |
| Failure [0 flpply for permil flod discharge continues after notice by POTW |  | $ 750 |  |

| Monitodng: |  |  |  |
| --- | --- | --- | --- |
| Failure to monilor aJ] pollLltants a~ required by permit (per parameter) |  |  |  |
| Failure to monitor all pollutants as required by permit (per parameter) |  | $100 |  |
| Failure to install monitoring equipment for more than 30 days (each day) | .$ 200 |  |  |
| Improper sampling or analysis (i.c. wrong lcchniques or methods) |  | $ 200 |  |
| Recurring failurc to monitor |  | $ 500 |  |
| POTW personnel denied entry to facility for inspection or sampling |  | $1,000 |  |

Sample results which exceed local or categorical limits will result in the assessment of administrative penalty or penalties as defined by the tables below.

Exceed Permit Limit By (Excluding pH violations)

| Flows | < 25% | 25-50% | 51-75% | 76-100% | > 100% |  |
| --- | --- | --- | --- | --- | --- | --- |
| < 1,000 GPD | NOV | NOV | $100 | $250 | $500 |  |
| 1,000-50,000 GPD | NOV | $100 | $200 | $400 | $650 |  |
| 10,001-50,000 GPD | NOV | $150 | $300 | $500 | $750 |  |
| 50,001-100,000 GPD | NOV | $200 | $400 | $750 | $900 |  |
| > 100,000 GPD | NOV | $250 | $500 | $900 | $1,000 |  |

pH Violations (Grab samples only)

|  |  | 6.0-6.49 |  | 5.5-5.99 |  | <5.5 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Flows |  | 10.01-10.50 |  | 10.51-12.50 |  | >12.50 |  |
| < 1,000 GPD |  | NOV |  | $100 |  | $200 |  |
| 1,000-10,000 GPD | NOV |  | $200 |  | $400 |  |  |
| 10,001-50,000 GPO |  | NOV |  | $300 |  | $600 |  |
| 50,001-100,000 GPD |  | NOV |  | $400 |  | $800 |  |
| > 100,000 GPD | NOV |  | $500 |  | $1,000 |  |  |

| Other permit violations: |  |  |
| --- | --- | --- |
| Waste slreams are diluted in lieu of rreauncnt | $500 |  |
| Failure to properly mainlCljn pretreatment faciJi(y | $500 |  |

| Failure [0 correct noncompliance or halt discharge: |  |  |
| --- | --- | --- |
| No harm to POTW | $500 $1,000 |  |
| Harm Lo POTW |  |  |

Repeated violations wilI be penalized, in addition to the administrative penalty, $200 for each event. This penalty will carryover and increase by $200 with every event in a series of recurrent violations.

Administrative penalty range increments increase per event, not per day.

Three successive administrative penalties will result in the issuance of a compliance schedule.

Fines will be levied for missed milestone dates during compliance schedule implementation.

Maximum administrative penalty per event is $5,000. Fines may be levied by the courts in

excess of these amounts.

Any or all administrative penalties may be waived Or modified per the discretion of the

Director of Public Safety and Service.

Any user who violates any provision of the City of Massillon Sewer Use Code ordinance may be liable for a fine(s) up to a maximum amount of $1 ,000 for each violation, with each day being separate violation. (925.51) (Ord. 61-2008. Passed 5-5-08,)

925.54 RECORDS RETENTJ ON

All dischargers subject to this chapter shall retain and preserve for no Jess than three years, any records, books, dOGuments, memoranda, reports, oorrespondence and any and all summaries thereof, relating to monitoring, sampling and chemical analyses made by or in behalf of a discharger in CORnection with its discharge. All records v'v'hich pertain to matters which are the subject of administrative adjustment or any other enforcement or litigation activities brought by the Superintendent pursuant hereto shall be retained and preserved by the discharger until all enforoement activities have concluded and all periods of limitation with respect to any and all appeals have expired.

Industrial Users subject to the reporting requirements of this ordinance shall

retain, and make available for inspection and copying, all records of information

obtained pursuant to any monitoring adivities required by this ordinance, any

additional records of information obtained pursuant to monitoring activities undertaken by the Industrial User independent of such requirements, and documentation associated with Best Management Practices established under Section 925.35(c). Records shall indude the date, exact place, method, and time of sampling, and the name of the person(s) taking the samples; the dates anafyses were performed, who performed the analyses; the analyHcal techniques or methods used; and the results of such ana~yses. These records shall remain available for a period of at least three years. This period shat! be a8.JItomatica~iy extended for the duration of any litigation concerning the industrial User or the City, or where the IndUistrial User has been specifocaHy nom~ed of a longer retenHol1 periods by the PlaInt Manager. (Ord, 69-1991. Passed 4-15-91.)

925.55 FEES

The City shall adopt charges and fees which include fees for monitoring, inspections,

and 5uPv'eillance procedures, fees for filing appeals, fees for permit application, and fees for revimving accidental discharge prooedures and construction. All money

received under the provisions of this chapter shall be oredited to the "Wastm·vater Treatment Re",'enue Fund" of the City.

(Ord. 69-1991. Passed 4-15-91.) The permit fee to be paid by the applicant for a City industrial Wastewater Discharge Permit shall be two hundred fifty dollars ($250.00); the permit essued under this chapter shall permit the holder of such permit to discharge waste into the City sewer system for a period of three years from the date of issuance. Such permit shan be renewable thereafter for 81 like three year period by payment of a renewal fee of two hundred fifty dollars ($250.00) and a fHing of a renewal application upon the form prOVided by the Plant Manager to amendment of the permit terms and conditions, or revocation or suspension of such permit as provided by this chapter.

925.56 RIGHT TO CONTRACT WITH INDUSTRY

(a) When determined by the Safety~Service Director and/or the P~ant Manager of the wastewater treatment system Plant that certain wastes and/or limitations heretofore prohibited by this chapter can be accepted wrthout detriment to the wastewater treatment system, the City may enter into special agreements WITh industry for the discharge of such wastes deemed acceptable. Pollutants can only be accepted in accordance with provisions of federal regulations.

(b) Any agreement entered into under the provisions of this section shall stipulate the type and amount of pollutant and/or conditions permitted thereunder, the conditions of permitting such discharges and the amount of surcharge in addition to applicable users charge and industrial cost recovery charges that will adequately compensate the City for all costs associated with collecting, pumping, treating and disposing of such discharges.

(Ord. 69-1991. Passed 4-15-91.)

925.60 BASELINE MONITORING REPORT

All Industrial Users, including new sources, that are subject to categorical pretreatment standards shall submit baseline monitoring report ("BMRs") to the Plant Manager. These reports shall supply basic information to ldentify the Industrial User, the characteristics of the User's discharge and the compliance status. A BMR must contain the following information:

| (a) | Name and address of the facility, including names of operator(s) and owner(s). |
| --- | --- |
| (b) | List of all environmental control permits held by or for the facility. |
| (c) | Brief description of the nature, average production rate, and SIC code for |

each of the operation (s) conducted, including a schematic process diagram which indicates points of discharge from the regulated process to the POTW.

(d) Flow measurement information for regulated process streams discharged to the Municipal system. Flow measurements of other wastestreams wlll be necessary if application of the combined wastestream formula is necessary.

(e) Identification of the pretreatment standards applicable to each regulated process and results of measurements of pollutant concentrations and/or mass. All samples must be representative of daily operations and results reported must include values for daify maximum and average concentration (or mass, where required). Where the flow of the regulated stream being sampled is less than or equal to 250,000 gallons per day, the Industrial User must take six samples within a two-week period. Where the flow of the stream is greater than 250,000 gallons per day, the Industrial User must take six samples within a tvvo week period. If samples cannot be taken immediately downstream from the regulated process and other wastewaters are mixed with the regulated process, the Industrial User should measure flows and concentrations of the other wastestreams sufficient to allow use of the combined wastestream formula.

(f) Statement of certification concerning compliance or noncompliance with the Pretreatment Standards.

(g) If not in compliance, a compliance schedule must be submitted with the BMR that describes the actions the user will take and a timetable for completing those actions to achieve compliance with the standard. This compliance schedule must contain specific increments of progress in the form of dates for the commencement and completion of major events, however, no increment of the schedule shall exceed nine months. Within fourteen days of each completion date in the schedUle, the Industrial User shall submit a progress report to the City indicating whether or not it complied with the increment of progress to be met on such date, and, if not, the date on which it expects to comply With this increment of progress and the steps being taken to return to the schedule.

(h) The baseline monitoring report does not apply to discharges not covered by categorical standards.

(i) Industrial Users shall submit BMRs to the Plant Manager within 180 days after the effective date on the applicable categorical standard, or within 180 days after the final decision on a category determination request, whichever is later.

U) New sources, and eXisting sources that become Industrial Users subsequent to the promUlgation of an applicable categorical standard, shall submit a baseline

monitoring report at least ninety days prior to commencement of the facility's discharge to a POTW. New sources may provide estimates for the information on production, flow, and the presence and quantity of regulated pollutants in its wastestream. New sources shall also provide information on the pretreatment equipment the new source proposes to install to meet applicable discharge limits.

(Ord.69-1991. Passed 4-15-1991.)

925.61 SIGNATORY REQUIREMENTS FOR INDUSTRIAL USER REPORTS AND CERTIFiCATIOf-JS

The application and repoJis required by Sections 925.32, 925.38, 925.39 and 925.60 shall include the following certification statement:

"I certify under penalty of law that this document and all attachments were prepared under my direction and supervision in accordance with a system designed to assure that qualified personnel properly gather and evaluate the information submitted. Based on my inquiry of the person or persons who manage the system, or those persons directly responsible for gathering the information, the information submitted is, to the best of my knOWledge and belief, true, accurate, and complete. I am aware that there are significant penalties for submitting false information, including the possibility of fine and imprisonment for knowing violations." and shall be signed as follows:

(a) By a responsible corporate office, if the Industrial User submitting the application and reports reqUired by Sections 925.32, 925.38, 925.39 and 925.60 is a corporation. For the purpose of this subsection, a responsible corporate officer means:

(1) A president, secretary, treasurer, or vice-president of a corporation in charge of a principal business function, or any other person who performs sImilar policy-or decision-making functions for the corporation, or

(2) The manager of one or more manUfacturing, production, or operation facilities employing more than 250 persons or having gross annual sales or expenditures exceeding $25 million (in seGoRd quarter 1980 dollars), if authority to sign documonts has been assigned or delegated to the manager in aGGordance "''lith corporate procedures. The manager of one Of more manufactudng, production, or operating facilities, provided that the manager is authorized to make management decisions which govern the operation of the regUlated facility, including having the expiicit or implidt duty of mClkiUlig major capital investment recommendations, and to initiate and direct other comprehensive measures to assure long~term environmental compliance with environmental laws and regulations; cail ensure that the necessary systems are estabHshed or actions taken to gather complete and accurate information for control mechanism requirements; and where authority to sign documents has been assigned or delegated to the manager in accordance with corporate procedures.

(b) By a general partner or proprietor if the Industrial User submitting the applications and reports required by Sections 925.32,925.38,925.39 and 925.60 is a partnership or sale proprietorship respectively.

(c) By a duly authorized representative of the individual designated in subsection (a) or (b) of this section if

(1) The authorization is made in writing by the individual described in subsection (a) or (b);

(2) The authorization specifies either an individual or a position having responsibility for the overall operation of the facility from which the Industrial Discharge originates, such as the position of plant manager, operator of a well, or well field superintendent, or a position of equivalent responsibility, or having overall responsibillty for environmental matters for the company; and

(3) The written authorization is submitted to the City.

(d) If an authorization under subsection (e) hereof is no longer accurate because a different individual or position has responsibility for the overall operation of the facility, or overall responsibility for environmental matters for the company, a new authorization satisfying the requirements of subsection (c) hereof must be submitted to the City prior to or together with any reports to be signed by an authorized representative.

Certification Statements

(1) Annual Certification for Non-Significant Categorical Industrial Users -A facility determined to be a Non-Significant Categorical industdal User by as described in Section 925.39 and pursuant to Section 925.39 must annually submit the following certification statement signed in accordance with the signatory requirements above. This certification must accompany an alternative report required roy the Plant Manager:

"Based on my inquiry of the person or persons directly responsible for mOlnag~ng compliance with the categorical Pretreatment Standards under

40 CFR _,rcertifythat,tothe bestofmyknOWledge and beliefthat

during the period from ,to 	[months,

days, year]. (a) The facility described as ---=:--~_--:---=--~_~--:---::---:~~,

[facility name] met the definition of a Non-Significant Categoricallndustri811

User as described in Section 925.39, (b) The facitity complied with all

app!icable Pretreatment Standards and requirements dudng this reporting

period; and (c) the facility never discharged more them 100 gallons of total

categorical wastewater on any given day during this reporting period, Thfs

compliance certification is based on the following information.

(Approximate justification may eodude water billing records~ production records, etc.)

(2) Certificatcon of Pollutants Not Present -Users that have an approved monitoring waiver based on Section 925.39 must certify on each report with the foUowing statement that there has been no increase in the pollutant in its waste stream due to activities aUhe Industrial User. Based on my inquiry of the person or persons directly responsible 40 CFR 403.6(a)(2)(ii) or the most recent applicable National Pretreatment Standard part(s) and shall be signed in accordance with the criterDa set forth in 40 CFR 403.12 (I)(i) -(4).

(3) 	Certification for Toxic Organics Management Plans -Users that have in force a City alpproved Total Toxic Organics Management Plan (TOMP) must certif~f on each report with the foHowing statement that there has been no dumping of contaminated toxic organics into the wastewater since the last monitoring report. (Based on my inquiry of the person or persons direcHy responsible for managing compliance with the pretreatment standard for total toxic organics (TTO), I certify that, to the best of my knOWledge and belief, no dumping of concentrated toxic organics into the wastewaters has occurred since filing the fast discharge monitoring report. ~ further certify that this facility is implementing the toxic organic management plan submitted to the control authority.)

(Ord. 69-1991, Passed 4-15-91.)

APPENDIX A ~ PRIORITY TOXIC POLLUTANTS

1. Acenaphthene

2. Acrolein

3. Acrylonitrile

4. Benzene

5. Benzidine

6. Carbon tetrachloride (tetrachioromethane)

7. Chlorobenzene

8. 1,2, 4 -Trichlorobenzene

9. Hexachlorobenzene

10. 1, 1 -Dichloroethane

11. 1, 2 -Dichloroethane

12. 1, 1, 1 -Trichloroethane

13. Hexachloroethane

14. 1, 1, 2 -Trichloroethane

15. 1, 1, 2, 2 -Tetrachloroethane

16. Chloroethane

17. Bis (2 -chloroethyl) ether

18. 2 -Chloroethyl vinyl either (mixed)

19. 2 -Chloronaphthalene

20. 2, 4, 6 -Trichlorophenol

21. Parachlorometa cresol

22. Chloroform (trichloromethane)

23. 2 -Chlorophenol

24. 1, 2 -Dichlorobenzene

25. 1, 3 -Dichlorobenzene

26. 1,4-Dichlorobezene

27. 3, 3 -Djchlorobenzidine

28. 1, 1 -Dichloroethylene

29. 1, 2 -Trans-dichloroethylene

30. 2,4-Dichlorophenol

31. 1, 2 -Dichloropropane

32. 1, 2 -Dichloropropene

33. 1,2 -Dichloropropylene

34. 2, 4 -Dimethylphenol

35. 2, 4 -Dinitrotoluene 36, 2, 6 -Dinitrotoluene

37. 1,2 -Dephenylhydrazine

38. Ethylbenzene

39. Fluoranthene

40. 4 -Chlorophenyl phenyl ether

41. 4 -Bromophenyl phenyl ether

42. Bis (2 -chloroisopropyl) ether

43. Bis (2 -chloroethoxy) methane

44. Methylene Chloride (dichioromethane)

45. Methyl Chloride (chloromethane)

46. Methyl bromide (bromomethane)

47. Bromoform (tribromomethane)

48. Dichlorobromomethane

49. Chlorodibromomethane

50. Hexachlorobutadiene

51. Hexachlorocyclopentadiene

52. Isophorone

53. Naphthalene

54. Nitrobenzene

55. 2 -Nitrophenol

56. 4 ~ Nitrophenol

57. 2, 4-Dinitrophenol

58. 4,6 -Dinitro-o-cresol

59. N -nitrosodimethylamine

60. N -nitrosodiphenylamlne

61. N -nitrosodi-n-propylamine

62. Pentachlorophenol

63. Phenol

64. Bis (2-ethylhexyl) phthalate

65. Butyl benzyl phthalate

66. Di-n-octyle phthalate

67. Di-n-buty! phthalate

67. Diethyl phthalate

68. Dimethyl phthalate

69. 1,2 -Benezanthracene (benzo (a) anthracene)

70. Senzo (a) pyrene (3,4 -benzopyrene) 71, 3,4 -BenzofJuoranthene (benzo (b) fluoranthene)

72. 11, 12 -Benzofluoranthene (benzo (k) fluoranthene)

73. Chrysene

74. Acenaphthylene

75. Anthracene

76. 1, 12 -Benezoperylene (benzo (ghi) perylene)

77. Fluorene

78. Phenanthrene

79. 1 2, 5, 6 -Dibenzanthracene (dibenzo (a, h) anthracene)

80. Indeno (1,2, 3-cd) pyrene (2, 3-o-phenylene pyrene)

81. Pyrene

82. Tetrachoroethylene

83. Toluene

84. Trjchloroethylene

85. Vinyl chloride (chloroethylene)

86. Aldrin

87. Dieldrin

88. Chlordane (technical mixture and metabolites)

89. 4,4 -DOT

90. 4,4-DOE (p, p-DDX)

91. 4,4-DOD (p, p-TDE)

92. Alpha-endosulfan

93. Beta-endosulfan

94. Endosulfan sulfate

95. Endrin

96. Endrin aldehyde

97. Heptachlor

98. Heptachlor epoxide (SHC hexachiorocyclohexane)

99. Atha -SHe

100. Beta -SHC

101. Gamma -SHe

102. Delta -BHe (PCB -polychlorinated biphenyls)

103. PCB -1242 (Arochlor 1242)

104. PCB -1254 (Arochlor 1254)

105.

106.

107.

108. 109,

110.

111.

112.

113.

114.

115.

117.

118.

119.

120.

121.

122.

123.

124.

125.

126.

PCB -1221 (Arochlor 1221) PCB -1232 (Arochlor 1232) PCB -1248 (Arochlor 1248) PCB -1260 (Arochlor 1260) PCB -1016 (Arochlor 1016) Toxaphene 2,3,7,8 -Tetrachlorodibenzo-p-dioxin

_____ 0

Antimony (Total) Arsenic Asbestos Berylium Cadmium Chromium Copper Cyanide, Total Lead Mercury Nickel Selenium Silver Thallium

Zinc