PRELIMINARY SITE PLAN

RYAN’S STEAKHOUSE

MASSILLON MARKETPLACE

ENGINEERING DEPARTMENT COMMENTS:

- Show elevations, existing and proposed topography on and next to site

- Call out all easements,  specifically the 10’ storm easement east of the property line

- Show locations of all utility hookups, electric, phone, gas, cable, sanitary w/grease trap, and storm sewer layout

- A storm water erosion control plan should be submitted, and followed

- Sanitary lateral may be run directly into manhole, or a wye placed onto the trunk line.  Your contractor will do this, however, he must be licensed with the city, obtain a permit, and have the connection inspected.

- No storm sewer permit is required, this can go into the catch basins or into the main line

- All other utilities have their own fees/permits and will need to be contacted directly

- Verify 8” sanitary line along Ohio 21, part of that was redone last year and 8” seems small for a trunk line, unless there are two lines there?

- Show details of driveway approaches

- Show traffic circulation pattern for parking, will they be striped?

- Sign heights/locations shall be coordinated with the building department

These are common questions and comments that come up during the site plan meeting.

If you have any questions, please call me at 330-830-1722.

____________________			_________________________

DATE 					JASON HAINES

TECHNICIAN