October 6, 2014

Hunters Point East Development LTD

1670 Akron Peninsula Road

Suite 101

Akron, OH  44313

ATTN: Dave Shepherd

RE:	Lake Ave. Site Plan Review

Mr. Shepherd:

The City of Massillon Engineering Department has reviewed the submitted site plan for the above referenced project.  The following comments have been compiled along with the attached set of red lined drawings need to be re-submitted for review.  In your re-submittal provide a response letter addressing each comment and three (3) revised sets plans and if applicable two (2) copies of supporting documentation.

GENERAL

- Label the property owners name and address on the site plan.

- Label the developers name and address on the site plan.

- Provide signature block for City Engineer as shown on the site plan checklist.

- Provide site plans which have been stamped by a licensed professional engineer.

- Show and label benchmarks used for this site.

SHEET 1  – SITE PLAN

- Label boundary information including pins found or set.

- Label right of way lines

- Label housing as temporary structures.

- Label which building is the sales office.

- Show and label front, side, and rear setbacks for a B-1 zoning district.

- Label the parcel numbers for the site.

- Label the total acreage of the site.

- Label owners, lot numbers, parcel numbers, and zoning for all adjoiners.

- Show limits of tree clearing.

- Show limits of disturbed areas and acreage of such.

- Label the dimensions of the driveway and curb cut.  Existing, and proposed.

- Show parking calculations on this sheet and supporting data.  (# employees, etc)

- Provide a swale along property lines and catch basins to catch as much surface and roof drainage as possible.  Set catch basin at NW corner of property and tie into storm system on Lake Avenue.  Show curb and street repair areas as needed.

- Show easement area and Volume / page for existing billboard sign.

- Show location for sanitary sewer connection.  Will new building addition have sanitary facilities?

- Show location for water, gas, electric, etc. connections on the plans.  Show locations of any street cuts or crossings which will be needed.

- Show location of existing fire hydrants.

- Add City of Massillon driveway detail to the plans.

- Add City of Massillon street repair detail to the plans.

- Provide an erosion/sediment control plan for this site.

- Provide a written request for variance of section 1183.03 of the zoning code to allow gravel paving and the reason for such.

- Label flood zone information on the site plan.

If you have any questions or need additional information please contact our office at 330-830-1722.

Respectfully,

KEITH A. DYLEWSKI, P.E., P.S.

CITY ENGINEER

Jason Haines

Engineering Technician

(Enclosures)