March 29, 2019

Greg Byrer

Thrasher Group

400 3rd Street SE

Suite 309

Canton, OH  44702

RE:	Ohio Wraps Site Plan Review

Mr. Byrer:

The City of Massillon Engineering Department has reviewed the submitted site plan for the above referenced project.  The following comments have been compiled for the site plan.  I did not create a redline mark up set.  In your re-submittal provide a response letter addressing each comment and three (3) revised sets of plans and if applicable two (2) copies of supporting documentation.

TITLE SHEET

- Add note and signature block for review of ESC plan……… “Approved by Stark Soil and Water District by letter dated ____________”.

SHEET 2 EXISTING CONDITIONS

- Label the names, lot numbers, and zoning for all adjacent parcels

- Label the recording instrument number for the 24 foot waterline easement

SHEET 3 SITE PLAN

- Label the front of the building.  Which road will site be addressed from?

- Correct the setback lines as 60 feet in an I-1 district.  If applying for a variance, a letter must be submitted requesting the variance and the accompanying hardship.  Setback along R-1 residential is 50 foot.

- Label the total area of proposed pavement

- Label the total disturbed area for the site including the utility installations.

- Note how parking spaces were calculated based on zoning code.  Number required / provided based on employee count, sf, etc.

- Provide a sight distance sheet showing sight distance at new driveway on south side of Finefrock road / SR 241

- Label the distance from the intersection to both driveway approaches.

- Label the dimensions of the concrete approaches.   Maximum commercial curb cut width is 30 foot.  If applying for a variance, a letter must be submitted requesting the variance and the accompanying hardship.

- Reduce the north driveway so that the existing sanitary manhole is not located in the approach area.

- Concrete approaches should be square to the edge of roadway and extend to the right of way line.  Show and label on the plans.

- Show and label site sign, lighting, and any fencing.

- Install screening along east property line (R-1 Residential) per Section 1183.03 of the zoning code

SHEET 4  GRADING PLAN

- Provide the flood zone and map number information on this sheet.

- Provide cross sections and detail drawing for each of the (3) detention structures including outlet structure details

- Eastern pond, swale?,  has no outlet ditch?  Pipe cannot dump onto east property line.

SHEET 6  UTILITY  PLAN/ SEWER PLAN

- Both proposed driveway culverts in the right of way are to be reinforced concrete pipe, a minimum of 12” diameter.  Label the lengths and note this on the plans.

- Move the proposed sanitary sewer lateral west, away from the culvert crossing in Oberlin Road,  as far as possible.

- Label the saw cut and repair areas for the sewer installation

- Show utility connections for gas, electric, phone service?

SHEET 7  DETAILS

- Provide City of Massillon driveway detail on the plans (see attached)

- Provide City of Massillon open cut restoration detail on the plans  (see attached)

If you have any questions or require additional information, please contact our office at 330-830-1722.

Respectfully,

Jason M. Popiel,  P.E., CPESC

CITY ENGINEER

Jason Haines

Engineering Technician

(Enclosures)