April 22, 2019

Hugh Thomas PE

BL Companies

3755 Boettler Oaks Drive, Suite G

Green, OH  44685

RE:	Dollar General Site Plan Review

Mr. Thomas:

The City of Massillon Engineering Department has reviewed the submitted site plan for the above referenced project.  The following comments have been compiled for your site.  In your re-submittal provide a response letter addressing each comment and three (3) revised sets plans and if applicable two (2) copies of supporting documentation.

GENERAL

- Provide approval letter from Stark Soil and Water Conservation District regarding approval of erosion control plans and a block on the title sheet for this.

- Provide approval letter from Aqua Ohio Water regarding approval of plans and a block on the title sheet for this.

- Provide a lighting plan showing all parking lot lighting and details for poles/fixtures

- Provide an auto turn sheet showing the semi truck turning radius for the new driveways and the 1st street NW / Cherry Road intersection.

SHEET EX01– EXISTING SURVEY / DEMO

- Label the lot numbers for site and all properties adjacent to the site.

- Label the zoning for this site and all properties adjacent to the site.

- Clearly label curb lines / edge of pavement for Cherry road and 1st Street.  Label the width of pavement on each street

- Show and label the pavement markings on Cherry Road.  Show location and termination of turn / stacking lane at signalized intersection.

- Show existing driveway locations, parking, buildings, etc across from both proposed driveways

SHEET SP01– SITE PLAN

- Remove existing/removed items from the site plan sheet, for clarity.

- Label the distance of the proposed driveways from the intersection and the highway ramps

- Remove ADA ramps at proposed driveway approaches.  Concrete sidewalk shall carry through the approach at grade, as per City driveway detail.

- Approaches are to be concrete, 9” thick.  Remove old curbing at the new approach areas.  Asphalt parking lot will begin at the back of sidewalk.  Show and label this on the site plans.

- Do not narrow the sidewalk around the highway sign.  Relocate sign out of new sidewalk area.

- Move the new approach away from the existing brick approach.

- Remove old driveway approach at SW corner of site and replace broken curbing.  Show and label this on the site plan.

- Show the existing driveway approach location (removed) and curbing to be replaced in this area

- Label the total building area and paved area of the site.

- Show location to sawcut and remove existing drive on north side of property.

SHEET GD01– PROPOSED GRADING PLAN

- Adjust the grading of the driveway approaches to match City sidewalk / driveway detail

- Label the flood zone / map / panel number on this sheet

- Provide gutter elevations at both driveways to show drainage.

SHEET UTO1– PROPOSED UTILITY PLAN

- Sanitary sewer shall be flushed, and video inspection provided to the City of Massillon Engineering department prior to a connection permit being issued.  If existing lateral is questionable, then a new lateral connection to the main sewer will be required.

- Sewer clean out is shown in middle of main driveway?  May need to add a manhole structure here with heavy lid.

SHEET LL01 02  – PROPOSED LANDSCAPE PLAN

- Provide screening along east side of parking lot against residential home.  Landscape or fencing

- Provide trees or shrubs along alley to separate from new driveway

- Fence detail should have 4x4 treated posts not 2x2 posts!

SHEET DN-01– DETAILS

- Add City of Massillon details for driveways, sidewalks, curbs, and roadway cut restoration  ( see attached )

STORM WATER MANAGEMENT REPORT

- Provide a Long-Term Maintenance Agreement for the maintenance of the on-site storm water management.

If you have any questions or require additional information, please contact our office at 330-830-1722.

Respectfully,

Jason M. Popiel,  P.E., CPESC

CITY ENGINEER

Jason Haines

Engineering Technician

(Enclosures)