June 3, 2019

Hammontree and Associates

5233 Stoneham Road

North Canton, OH  44720

ATTN: Joshua Renicker PE

RE:	Villa Sole Development Site Plan Review

Mr. Renicker:

The City of Massillon Engineering Department has reviewed the submitted site plan for the above referenced project.  The following comments have been compiled for the site plan.    In your re-submittal provide a response letter addressing each comment and three (3) revised sets of plans and if applicable two (2) copies of supporting documentation.

TITLE SHEET

- Clearly label the site on the vicinity map. Label Sherman Circle on the map as it is called out in the description but not shown….

- List the total proposed public street, sanitary sewer, water line, etc  on this sheet.

- A final plat and easement plat will be required prior to construction.

- Dedication of remaining right of way areas for Hills and Dales Road will be required with the final plat.  Adjustment to setback lines may be needed.

- Provide a copy of the geotechnical report for this site, as listed on sheet C8.1

- Clearly show the 30% open space required in Section 1159 PUD developments.

- Label Via Luna circle as “private road” on all sheets and section view.

SHEET 2.1 EXISTING CONDITIONS

- Show and label the existing sanitary sewer in the SW corner of the lot.  This sewer runs across the area of the new headwall and then north on or along the property line and needs to be accurately shown

- What are the asterisks referencing on the east property line items?

SHEET 3.1 SITE PLAN

- Show and label the existing right of way width for Hills and Dales Road

- Show and label the existing pavement width for hills and Dales Road

- Label the existing pavement width for Wild Ridge Circle

- Show and label the zoning areas and the line between residential lots and PUD lots.

- Label subdivision lots 1-4

- Label the total area of proposed pavement

- Label the distance from the intersection of Sherman Circle to the new driveway approach

- Label the dimensions of the concrete approaches.   Maximum approach width is 30 foot.  Approach is to be 9” concrete to the R/W line.   Label width of curb cut.

- Do not show curbing as removed, it will be sawcut.  The existing curb cut under the guardrail (!! ) will need to be removed and replaced.   The guardrail and impact corners will remain in their current location and will not be moved.

- Public road Wild Ridge cul de sac shall be curbed per City section detail. Revise storm catch basins accordingly on the plans.  Inlet grate to be East Jordan 7505

- Show and label site sign location.  Should not be in R/w areas of Hills and Dales Road.

SHEET 4.1  UTILITY  PLAN/ SEWER PLAN

- Label sanitary manhole 8 as existing, are you proposing to replace?  Provide detail of  bottom inverts and how you propose to tie into manhole from the east, against existing flow.

- Raise manhole 8 T/C if needed to match pond embankment grading

- Label existing county sewer laterals in Wild Ridge as “not used”

- Label the saw cut and repair areas for the water line  installation in Hills and Dales Road.

- Label the required separation between sewer, water, storm lines on the plan sheet.

- Provide utility conflict chart where storm / sanitary lines cross

- YD 1 to YD2 should be 10 inch pipe not 8 inch

- Show 4 inch perforated pipe in plan view

SHEET 5.1,6.1  GRADING PLAN / DETENTION POND

- Provide rock channel protection at emergency spillway

- Provide rock channel protection along west embankment  of detention pond if needed. What is high water elevation of the ditch?

- Provide drainage swale along west property line at Wildridge cul de sac

The house location does not allow room for grading to be done…..

- Rear yard swale to pond needs erosion control and details

- Label slopes of pond banks and proposed hill to north and east…

- Outlet elevation is 1052 and bottom is listed as 1048?  How will pond drain?  Is this to be a wet pond?

SHEETS 7.1 -7.3  DETAILS

- Provide City of Massillon driveway detail on the plans (see attached)

- Provide City of Massillon open cut restoration detail on the plans  (see attached)  REMOVE the existing trench detail

- Provide City of Massillon sanitary details to the plans.  Drop manholes, castings, etc.

SHEET 8.1 SECTIONS

- Provide City of Massillon roadway section detail for Wild Ridge Ave  including curb/ gutter and underdrains.

- Label Wild Ridge Ave section as “public roadway”

- Label Via Luna circle section as “private roadway”

- Provide sawcut/butt joint detail for tie-in at existing Wild Ridge pavement

SHEET 8.3  and 8.8 PROFILES

- Show the existing sewer manhole in Wildridge ave and the existing pipes.  If laterals are cut and not used, they should be capped off with concrete.  Not this on the plans and profile sheet.

- Show manhole casting to be adjusted to new pavement grade and how this will be achieved.

- Manhole #3 needs changed.  Should not have a sump.

- Grading from manhole 3 to 4 needs changed to allow minimum 4 foot of cover.   As shown, the pipe is exposed.

SHEET 8.5 INTERSECTION DETAILS

- Curve data for  16, 17, 18 are missing from the table.  Does the cul de sac meet City requirement for a public road?

- Monument box and pin will be required at center of cul de sac.  Label and add details.

- Label the dimensions of the concrete approach.

STORM WATER / DETENTION

- Need Long Term Maintenance agreement for pond

- Ensure 25 year HHL calcs are correct with a 25 year tailwater elevation shown in report ( 25 year pond elevation is tailwater elevation)?

- Calcs show a 1.9-inch orifice at elevation 1052.  Plans show 2.25-inch orifice?

If you have any questions or require additional information, please contact our office at 330-830-1722.

Respectfully,

Jason M. Popiel,  P.E., CPESC

CITY ENGINEER

Jason Haines

Engineering Technician

(Enclosures)