September 26, 2021

Keith Dylewski, PE

CivPro Engineering

4450 Belden Village ST

Suite 800

Canton, OH  44718

RE:	Joe Berens  Site Plan Review

Mr. Dylewski:

The City of Massillon Engineering Department has reviewed the submitted site plan for the above referenced project.  The following comments have been compiled and need to be revised and re-submitted for final review.  In your re-submittal provide a response letter addressing each comment and three (3) revised sets plans and if applicable two (2) copies of supporting documentation.

GENERAL

- Label the total disturbed area.  Is submittal necessary to Stark Soil and water district?

- Label the intended use of the property and buildings.

- Label the lot number as Out Lot 1231.  This area is currently in the process of replatting

- Label the  lot numbers, owners, and zoning for all surrounding properties.

- Show location of end of pavement and fence/gate across James Ave

- Label the address of the existing building on the plans

- Label the dimensions of the existing building on the plans

- This does not appear to be a building addition as the title sheet implies.  Correct the title on sheet 1

- Label the total area of structures, pavement/gravel, and landscape/grass for the site.

- Label total employee parking required for the business use and total parking spots provided.

- Label the pipe size, depth, casting elevations for the storm sewer in James Ave. Is this drainage system adequate to handle development from this site?

- Are utilities provided to the new building?  Show location

- Show location of existing septic system and leach fields

- Where is the fuel island reclocated to?  Show this on the plans.

- Provide the flood zone and map number information.

- Show and label site sign, lighting, and any fencing.

If you have any questions or require additional information, please contact our office at 330-830-1722.

Respectfully,

Jason M. Popiel,  P.E., CPESC

CITY ENGINEER

Jason Haines

Engineering Technician