September 27, 2021

Kevin Glick

18770 Withrich Road

Dalton, OH  44618

RE:	Glick Allotment Plan Review #1

Mr Glick:

The City of Massillon Engineering Department has reviewed the submitted sketch plan for the above referenced project.  The following comments have been compiled and need to be revised and re-submitted for final review.  In your re-submittal provide a response letter addressing each comment and three (3) revised sets plans and if applicable two (2) copies of supporting documentation.

GENERAL

- Please provide a set of construction drawings signed and stamped by a professional engineer licensed in the State of Ohio. Proposed grading and utility work shown will require following the subdivision regulations.

- Provide a name for the subdivision.  Raynell allotment already exists to the west.

- Provide plans on 24x36 sheets.

- Provide a proposed grading plan, showing existing and proposed contours and topography.

- Show limits of tree clearing.

- Show the existing home and buildings/driveways on the property and location.

- Label the existing lot number, owner, and zoning  for the site

- Label the existing lot number, owner, and zoning for all adjacent properties.

- Show adjacent home locations and addresses.

- Show and label the corporation line for the City of Massillon

- Label pavement width and show edge of pavement for all roadways

- Provide existing and proposed utility information. Top casting/flowline for all manholes and catch basins.

- Sanitary main does not extend north along 29th Street as drawn.  Sanitary sewer stops at the existing driveway to the house.  Show location  and depth of manhole on the plans

- Label sewer laterals as proposed and as  6- inch pipe.  Provide a detail of how these laterals will be cut into the existing lines with an 8x6 wye and Ferncos.  Saddle connections are not permitted

- Show saw cut and roadway repair areas for all utility cuts

- Show the existing creek and culvert crossing at 29th Street

- Show the ditch location along 29th Street flowing north to ditch? Show culvert at existing driveway?  Ditch may need to be re graded to take water from new storm sewer as part of the improvements

- Label elevations of new catch basins. Label offset from pavement  and property line for new storm sewer.

- Show locations of driveways and new culverts.  12-inch minimum size

- Provide erosion control plan and details as needed.

- Show proposed lot dimensions

- Label north lot as “non-buildable” if that is your intention

- Show setbacks for all new lots

- Show new home locations and finish floor elevations

- Include the necessary details for construction on the plans. See attached for City roadway repair detail

If you have any questions or require additional information, please contact our office at 330-830-1722.

Respectfully,

Jason M. Popiel,  P.E., CPESC

CITY ENGINEER