March 16, 2023

Keith Dylewski, PE

CivPro Engineering

4450 Belden Village ST

Suite 800

Canton, OH  44718

RE:	Walnut Woods Site Plan Review

Mr. Dylewski:

The City of Massillon Engineering Department has reviewed the submitted site plan for the above referenced project.  The following comments have been compiled and need to be revised and re-submitted for final review.  In your re-submittal provide a response letter addressing each comment and three (3) revised sets plans and if applicable two (2) copies of supporting documentation.

GENERAL

- Change signature block for “Approved by Massillon Engineering Department” and remove Jason Popiel

- Clearly show the site location, identify lot, parcel number, acreage of the site.

- Label the existing zoning of the site

- Label existing sidewalk, curb, edge pavement, etc

- Label the developer name, contact information

- Label the use of the proposed development

- Number the proposed buildings, label how many units in each building

- Label dimensions of buildings

- Label actual setbacks of each building

- Label distance between buildings

- Show summary of parking required/provided on the site plan.  If a variance is needed, it must be requested in writing, with a letter explaining the reason that a variance is required.

- Show summary of SF building area, paving area, landscape area on the site plan

- Show limits of site clearing

- Show limits of flood plain area along Wetmore Creek

- Provide site lighting/photometric plan

- Provide plan/profile sheet for sanitary sewer

- Provide details for core drill/ invert connection into existing manhole

- Provide details and profile for creek crossing--depth? encasement? bore?

- Provide details for sanitary drop manhole (see attached)

- Provide EPA Permit to Install for sanitary sewer

- Provide Easement dedication plat for 40-foot-wide sanitary sewer easement along north property line of the site (sanitary trunk sewer line)

- Provide 8-inch diameter sewer pipe between manholes

- Show a 6-inch diameter sewer lateral pipe to each building

- Move pond outlet pipe (DP19) away from the sanitary sewer trench

- Provide a headwall at DP19 storm pipe outlet, swale to creek?

- Show rock channel protection at all storm outlets and provide calculations

- Label distance from proposed driveway to existing 13th St. intersection

- Clearly show proposed driveway apron-width of curb cut, replacement of sidewalk areas, etc

- Show location of gas service lines

- Show any road cut areas and note replacement by developer

- Add City of Massillon driveway detail to the plans

- Add City of Massillon roadway trench detail to the plans.

- Add City of Massillon sidewalk/curb details to the plans

- Add City of Massillon sanitary details to the plan

- See attached storm sewer calculation red marks. The cumulative area (and therefore the cumulative time and C*A) is incorrect. The addition should be completed following the flow path of the pipe network. Red marks show correct values. Needs recalculated for Area, CA and Time.

- Storm sewer calculations need to show T/G elevations, so the cover information is corrected.

- On the Drainage Structures Drainage Map, please label/text the area (in acres) and weighted C value for each basin.

- Regarding hydrograph 4P in the stormwater routing calculations, the 1.2” permanent orifice is not accounted for as shown on sheet 12/18.

- Need a Long-Term Maintenance Agreement (maybe a SWCD comment)

- Regarding the Storm sewer structure data on page 6/18:

  - DS-H and DS-I appear to have their invert data switched around.

  - The 6” invert shown under DS-G should be 985.99 (at least per the storm sewer calculation sheet)

  - The 6” invert shown under DS-C should be 989.09 (at least per the storm sewer calculation sheet)

  - The 10” invert should be labeled as a 4” under DS-4

- On sheet 6, the slope of DP-14 under the storm sewer pipe table should be 3.95% not 5.55% (at least per the storm sewer calculation sheet)

- On sheet 4, the keynote #4 should provide the size and invert of the tee.

- Please clarify in the narrative how a 7+ acre site has no flow through the two-year storm. Is the entire site infiltrated? This appears to hurt your allowable outflow, which will not be zero when comment #4 is addressed. Feel free to call to discuss.

If you have any questions or require additional information, please contact our office at 330-830-1722.

Respectfully,

Gregory A .McCue

Civil Engineer IV

Jason Haines

Engineer Technician