September 15, 2023

Jennifer Schumacher PE

Hammontree and Associates

5233 Stoneham Road

North Canton, OH  44720

RE:	Dairy Queen Site Plan Review

Mrs Schumacher:

The City of Massillon Engineering Department has reviewed the submitted site plan for the above referenced project.  The following comments have been compiled and need to be revised and re-submitted for final review.  In your re-submittal provide a response letter addressing each comment and three (3) revised sets plans and if applicable two (2) copies of supporting documentation.

GENERAL

- Provide a replat/split to create the new Out Lot

- Clearly label the right of way line, and width,  for Erie Ave/SR 21

- Label the access road as “private”

- Label total building area, and pavement areas for the site

- Provide easement documents for sanitary sewer? along the east side of the site.  I cannot find any record of sanitary sewer here.

- Label distance of new driveways from existing signal intersection

- Label dimensions of curb cuts and drive approaches

- Dumpster enclosure is located in front setback?

- Poles for site lighting are located in Edison easement.  Conflict with overhead lines?

- Move sidewalk/patio out of sanitary sewer easement area

- Move pylon sign and foundation out of sanitary sewer easement area

- Provide a detail for sewer stack (ductile iron tee) and call out on the plans

- Existing catch basin #1 is probably a yard basin (nyloplast) and will need to be replaced with a precast structure and traffic bearing grate

- Clearly label the dominion easement area and locate new gas line in this area.  It is not clear on the plans

- Provide a plumbing plan /fixture count for the new building.

- Provide a lighting plan and photometric plan for the site

If you have any questions or require additional information, please contact our office at 330-830-1722.

Respectfully,

Alex Pitts PE

City Engineer

Jason Haines

Engineer Technician